CITY OF TUSCALOOSA

STORM-WATER ASSESSMENT

1913 Hurricane Creek Railroad Trestle destroyed by April 27, 2011 tornado

(Photo: aerial footage courtesy of Southwings, 2011)

GEOLOGICAL SURVEY OF ALABAMA

Berry H. (Nick) Tew, Jr.

State Geologist

CITY OF TUSCALOOSA STORM-WATER ASSESSMENT

OPEN-FILE REPORT ___

A REPORT TO THE

CITY OF TUSCALOOSA

FINAL REPORT

##### By

Marlon R. Cook

Mac McKinney

and

Alana L. Rogers

Submitted in partial fulfillment of contract with the City of Tuscaloosa.

Tuscaloosa, Alabama

2013

Table of Contents

Introduction		1

Acknowledgments		3

Monitored watersheds			3

Land use and impervious surfaces		3

Water quality monitoring		13

Chemical and physical parameters		13

Stream discharge		13

Stream temperature		15

Specific conductance		16

pH		18

Dissolved oxygen		20

Biochemical oxygen demand		21

Chemical oxygen demand		21

Total organic carbon		22

Total dissolved solids		23

Turbidity		24

Constituent concentrations and loading in project streams		27

Nutrients in project streams		27

Ammonia		27

Nitrate		28

Phosphorus		29

Bacteria			31

Metallic constituents		32

Inorganic nonmetallic constituents		35

Organic constituents		36

Sedimentation 			38

Sediment loads transported by project streams		39

Streams flow conditions		43

Suspended sediment		46

Bedload sediment		50

Total sediment loads		55

Summary and conclusions		58

References cited		59

Appendix		66

Illustrations

Figure 1. Impervious surfaces percentages for Tuscaloosa storm-water

assessment sites		12

Figure 2. Changes in stream form associated with urbanization		14

Figure 3. Stylized relationship between percentage of  imperviousness

and receiving stream impact		15

Figure 4. Measured specific conductance and discharge values for monitoring  site 3		17

Figure 5. Measured specific conductance and discharge values for monitoring site 9		17

Figure 6. Measured pH values for monitoring site 2		19

Figure 7. Measured pH values for monitoring site 7		19

Figure 8. Measured pH values for monitoring site 8		19

Figure 9. Measured TOC values for monitoring site 1-10		23

Figure 10. Measured turbidity and discharge for monitoring site 4		25

Figure 11. Measured turbidity and discharge for monitoring site 6		26

Figure 12. Measured turbidity and discharge for monitoring site 11		26

Figure 13. Measured turbidity and discharge for monitoring site 12		26

Figure 14. Measured turbidity and discharge for monitoring site 13		27

Figure 15. Measured concentrations of ammonia (NH3 as N) for sites 1-10		28

Figure 16. Measured concentrations of nitrate (NO3 as N) for sites 1-10		29

Figure 17. Measured concentrations of total phosphorus for sites 1-10		30

Figure 18. Measured concentrations of fecal coliform for sites 1-10		32

Figure 19. Measured concentrations of phenol for sites 1-10		37

Figure 18. Stream conditions downstream from site 13		39

Figure 21. Geologic map of Tuscaloosa County showing Fall Line		41

Figure 22. Critical stream flow velocity determined from measured velocities at site 12, Rum Creek at Highway 69		42

Figure 23. Critical stream flow velocity determined from measured velocities at site 5, Cypress Creek at Highway 69		43

Figure 24. Measured discharge at site 19, unnamed tributary to the Black Warrior River at Rice Mine Road		44

Figure 25. Measured discharge at site 13, Cypress Creek at Highway 82		44

Figure 26. Measured discharge at site 4, Cribbs Mill Creek at Kauloosa Avenue		44

Figure 27. Measured discharge at site 6, unnamed tributary to Cottondale Creek at JVC Road		45

Figure 28. Linear relationship between suspended sediment and stream discharge at site 17, Cribbs Mill Creek at 2nd Avenue East		47

Figure 29. Exponential increase in suspended sediment transport relative to discharge at site 10, unnamed tributary to the Black Warrior River at Indian Hills Country Club		48

Figure 30. Decreasing rate of suspended sediment transport relative to increasing discharge at site 8, unnamed tributary to Hurricane Creek near Summerfield Subdivision		48

Figure 31. Contrasting rates of suspended sediment transport relative to discharge at site 11, unnamed tributary to Moody Swamp at Fosters Ferry Road			49

Figure 32. Estimated annual suspended sediment loads for sites in the Tuscaloosa storm-water assessment		50

Figure 33. Estimated normalized annual suspended sediment loads for sites in the Tuscaloosa storm-water assessment		50

Figure 34. Critical stream flow velocity determined from measured velocities and bedload at site 5, Cypress Creek at Highway 69		52

Figure 35. Measured stream flow velocity and bedload transport rates at site 14, Cypress Creek at Cypress Creek Avenue		52

Figure 36. Measured stream flow velocity and bedload transport rates at site 12, Rum Creek at Highway 69		52

Figure 37. Estimated total annual bedload transport rates for Tuscaloosa storm-water assessment sites		53

Figure 38. Estimated normalized annual bedload transport rates for Tuscaloosa storm-water assessment sites		54

Figure 39. Estimated suspended and bed sediment loads for Tuscaloosa storm-water assessment sites		55

Figure 40. Estimated normalized suspended and bed sediment loads for Tuscaloosa storm-water assessment sites		56

Figure 41. Estimated total annual sediment loads for sites in the Tuscaloosa storm-water assessment		56

Figure 42. Estimated normalized total annual sediment loads for sites in the Tuscaloosa storm-water assessment		57

Figure 43. Total annual sediment loads for selected streams in Alabama		58

##### Tables

Table 1. Measured area in square miles for the Tuscaloosa storm-water assessment

sites		4

Table 2. Land use data for Tuscaloosa storm-water assessment sites		10

Table 3. Developed areas for the Tuscaloosa storm-water assessment sites		11

Table 4. Impervious surface data for the Tuscaloosa storm-water assessment sites 		12

Table 5. Measured temperature values for the Tuscaloosa storm-water assessment sites		15

Table 6. Measured specific conductance values for the Tuscaloosa storm-water assessment sites		16

Table 7. Measured pH values for the Tuscaloosa storm-water assessment sites		18

Table 8. Measured dissolved oxygen (DO) for the Tuscaloosa storm-water assessment sites		20

Table 9. Measured biochemical oxygen demand (BOD) for sites 1-10		21

Table 10. Measured chemical oxygen demand (COD) for sites 1-10		22

Table 11. Measured total dissolved solids (TDS) for sites 1-20		23

Table 12. Measured turbidity in nephelometric turbidity units (NTD) for sites 1-20		24

Table 13. Measured concentrations of ammonia (NH3 as N) for sites 1-10		28

Table 14. Measured concentrations of nitrate (NO3 as N) for sites 1-10		29

Table 15. Measured concentrations of total phosphorus for  sites 1-10		30

Table 16. Measured concentrations of fecal coliform for sites 1-10		32

Table 17. Measured concentrations of metallic constituents detected in water samples for Tuscaloosa storm-water assessment sites 1-5		33

Table 18. Measured concentrations of metallic constituents detected in water samples for Tuscaloosa storm-water assessment sites 6-10		34

Table 19. Measured concentrations of inorganic nonmetallic constituents detected for the Tuscaloosa storm-water assessment sites 1-5		36

Table 20. Measured concentrations of inorganic nonmetallic constituents detected for the Tuscaloosa storm-water assessment sites 6-10		36

Table 21. Measured concentrations of phenol for sites 1-10		37

Table 22. Summary data for stream discharge from project watersheds		45

Table 23. Summary statistical data for sediment loads estimated for the Tuscaloosa storm-water assessment		57

Plates

Plate 1. Monitoring sites and watersheds for the City of Tuscaloosa storm-water assessment.

Plate 2. Land-Use/Land-Cover for the City of Tuscaloosa storm-water assessment.

Plate 3. Nitrate concentrations for City of Tuscaloosa storm-water assessment watersheds.

Plate 4. Phosphorus concentrations for City of Tuscaloosa storm-water assessment watersheds.

Plate 5. Total sediment loads for the City of Tuscaloosa storm-water assessment watersheds.

CITY OF TUSCALOOSA STORM-WATER ASSESSMENT

by

Marlon R. Cook, Mac McKinney, and Alana L. Rogers

INTRODUCTION

This document is the final report for work completed during 2004-2005 and 2012-2013 on the City of Tuscaloosa Storm-Water Assessment as provided for under terms of 2004 and 2012 agreements between the Geological Survey of Alabama (GSA) and the City of Tuscaloosa. The report presents information concerning storm water runoff in 20 subwatersheds that drain stormwater from the City of Tuscaloosa in the Black Warrior River watershed in Tuscaloosa County, Alabama. The analytical data included in this report were compiled during the period April 2004 to May 2005 (Cook and others, 2005) and from December 2012 to June 2013.

The City of Tuscaloosa is in a unique hydrologic setting, which is determined by the underlying geology of the area. Tuscaloosa lies along the Fall Line, the boundary between hard rocks of Paleozoic Age underlying the northern portion of the city and unconsolidated Coastal Plain sediments that underlie the southern portion of the city. Geologic structures in the Pottsville Formation and the Valley and Ridge Physiographic Province southeastward from the city influence the flow of the Black Warrior River and its tributaries. Most of the groundwater and surface water in the city south of the Black Warrior River flows southwestward into Moody Swamp before draining into the river. A relatively small amount of water drains northward directly into the river and eastward into Hurricane Creek, which flows into the Black Warrior River. Water in the city north of the river drains southward into the river. Another unique feature of the 2012-2013 monitoring is documentation of water quality impacts related to the April 2011 tornado that devastated parts of the city and resulted in widespread land-use change.

Urban storm water has two primary characteristics that differ from rural runoff:  1) increased volume and velocity of runoff, and 2) large concentrations of contaminants. Both characteristics are directly related to development in urban and urbanizing areas. Together, these characteristics cause changes in hydrology and water quality that result in a variety of problems including habitat loss, increased flooding, decreased aquatic biological diversity, and increased sedimentation and erosion. The general decline in water quality in the urban environment may also adversely affect the quality of shallow groundwater and downstream surface water and may adversely affect the future quality of public water supply sources.

Urban runoff pollutants are many and varied depending on land uses and pollutant sources present in an urban area. Typically, loadings of urban pollutants are greatest from industrial and commercial areas, roads and freeways, and higher density residential areas. Although sources of specific pollutants may vary widely in urban areas, motor vehicles are recognized to be a major source of pollutants, contributing oils, greases, hydrocarbons, and toxic metals.

Major categories of urban pollutants include sediments, nutrients, microbes, and toxic metals and organics. Sediment concentrations in urban runoff are particularly problematic because of their ubiquitous nature, and the fact that many other pollutants occur in association with sediment particles. Sediment loadings occur primarily from soil erosion and runoff from construction sites in urban areas.

Sources of nutrients, such as nitrates and phosphates, include chemical fertilizers applied to landscaped areas, lawns, and gardens, failed septic systems, soil erosion, and atmospheric deposition. Excessive nutrients in urban runoff can stimulate algal growth and cause nuisance algal blooms. Urban runoff may also contain high levels of organic matter that can lead to depleted oxygen levels in water and sediment when it decomposes.

Microbes include hundreds of different kinds of bacteria, protozoa, and viruses that are ubiquitous in the natural environment. Many are beneficial, while others can cause diseases in aquatic biota and illness or even death in humans. Some types of microbes are pathogenic (e.g., Giardia spp.), while others indicate a potential risk for water contamination (e.g., fecal coliform bacteria). Microbes are almost always found in high concentrations in urban stormwater, but are highly variable in nature and very difficult to eliminate. Primary sources of microbes include failed sewer or septic systems and waste products from pets, birds, and wild mammals commonly found in urban areas.

Toxic pollutants commonly found in urban runoff include trace metals such as lead, copper, zinc, and organic compounds including oils, grease, phthalates, and chlorinated hydrocarbons. Sources of toxins include the breakdown of metal products, vehicle fuels and fluids, vehicle wear, industrial processes, and the use of industrial and household chemicals such as paints, preservatives, and pesticides. Trace metals and organic compounds may be highly toxic to aquatic organisms and can bioaccumulate in fish and shellfish.

The stormwater assessment presented in this report was designed to characterize the physical and geochemical character of stormwater in the city of Tuscaloosa. The assessment consists of comprehensive geochemical analyses of single, high-flow water samples collected from each of 10 key watersheds that drain storm water from the city. The assessment also contains estimates sediment transport rates (including annual sediment loads) for 20 watersheds that drain the majority of stormwater from the city.

###### ACKNOWLEDGMENTS

The Geological Survey of Alabama acknowledges those individuals whose participation and cooperation made this project possible: The Honorable Walt Maddox, Mayor of the City of Tuscaloosa; Mr. David Griffin, City Engineer; Mr. Chad Christian, Storm Drainage Engineer; and Mr. Josh Yates, Engineering Environmental Compliance Coordinator, whose support was essential for the design and completion of the project.

###### MONITORED WATERSHEDS

The City of Tuscaloosa Storm Water Assessment project area is approximately 38 square miles. Nineteen monitoring sites were established in 19 subwatersheds, varying in size from approximately 0.3 to 6.2 square miles (mi2) (table 1).  The project area and monitored watersheds are shown on plate 1. The monitoring sites were on three tributaries to Hurricane Creek in the eastern area of the city, four tributaries that flow directly into the Black Warrior River, two sites on Cribbs Mill Creek, a tributary of Moody Swamp, and ten additional sites on tributaries of Moody Swamp. All monitoring sites and their associated subwatersheds, with the exception of site 9, were kept consistent with the previous storm water assessment plan from 2005. Site 9 was excluded from this study due to restricted access to the sampling location. Names of monitored streams and site numbers are given in table 1. Monitoring site locations are shown on plate 1.

Table 1.—Measured area in square miles for the Tuscaloosa stormwater assessment sites.

| Monitoring site numbers | Monitored stream name and site location | Monitored watershed area ( mi2) |
| --- | --- | --- |
| 1 | Unnamed tributary to the Black Warrior River at Jack Warner Parkway (U.S. Army Corps of Engineers facility) | 0.384 |
| 2 | Unnamed tributary to Moody Swamp at 29th Street | 0.882 |
| 3 | Unnamed tributary to Moody Swamp at MLK Drive | 2.221 |
| 4 | Cribbs Mill Creek at Kauloosa Avenue | 1.854 |
| 5 | Cypress Creek at Highway 69 | 0.867 |
| 6 | Unnamed tributary to Cottondale Creek at JVC Road | 4.559 |
| 7 | Unnamed tributary to Hurricane Creek at Keenes Mill Road | 1.759 |
| 8 | Unnamed tributary to Hurricane Creek near Summerfield subdivision | 1.239 |
| 10 | Unnamed tributary to the Black Warrior River near Indian Hills Country Club | 1.015 |
| 11 | Unnamed tributary to Moody Swamp at Fosters Ferry Road | 0.617 |
| 12 | Rum Creek at Highway 69 | 6.204 |
| 13 | Cypress Creek at Highway 82 | 0.970 |
| 14 | Cypress Creek at Cypress Creek Ave | 1.491 |
| 15 | Cypress Creek at Diamond Ridge Lane near Spring Hill Lake | 0.842 |
| 16 | Unnamed tributary to Cribbs Mill Creek at 2nd Avenue East | 3.337 |
| 17 | Cribbs Mill Creek at 2nd Avenue East | 2.981 |
| 18 | Unnamed tributary to Cribbs Mill Creek at Hargrove Road | 1.935 |
| 19 | Unnamed tributary to the Black Warrior River at Rice Mine Road | 1.136 |
| 20 | Unnamed tributary to the Black Warrior River at Rice Mine Road | 3.308 |

* Cypress Creek drainage area from headwaters

LAND USE AND impervious surfaces

Land use is directly correlated with water quality, hydrologic function, ecosystem health, biodiversity, and the integrity of streams and wetlands. When natural landscapes are converted to urban areas, the ground is changed from porous ground to impervious surfaces such as roads, sidewalks, parking lots, and buildings. Impervious surfaces or impervious cover is defined as all hard surfaces of the watersheds that are impermeable to rainfall infiltration into the subsurface. Increases in imperviousness lead to higher rates and volumes of storm water runoff which can have devastating effects on local hydrology including: increased flooding and streambank erosion, degraded aquatic habitat, reduced groundwater recharge, and increased surface-water pollution. Subwatersheds within the City of Tuscaloosa project area are dominated by urban development which is characterized by large areas covered with impervious surfaces.

Land use classification within each watershed was calculated from the USDA, National Agricultural Statistics Service, 2012 Alabama Cropland Data Layer (NASS CDL) raster dataset. In the 2005 report, both the land use and impervious surface was calculated from the 2001 Multi-Resolution Land Characteristics Consortium- National Land Cover Database (MRLC NLCD) raster dataset. The most recent form of the MRLC NLCD dataset is from 2006. To employ more current methods of land use classification within this report, the 2012 NASS CDL raster dataset was utilized for spatial analyses. Both the NASS CDL and the MRLC NLCD are comparable in that they have a ground resolution of 30 meters and include the 2006 NLCD derived land use classifications. The CDL is produced using satellite imagery from the Landsat 5 TM sensor, Landsat 7 ETM+ sensor, the Spanish DEIMOS-1 sensor, the British UK-DMC 2 sensor, and the Indian Remote Sensing RESOURCESAT-1 (IRS-P6) Advanced Wide Field Sensor (AWiFS) collected during the current growing season (USDA 2012). All crop-specific classes were merged into a singular land use class titled “agricultural” to keep a consistent classification system with the 2005 report. Land use in the project area was subdivided into six classified groups defined as developed, forested, agricultural, grassland/shrub/scrub, wetlands, and open water (table 2). Land use is shown on Plate 2.

Figure 1.—Changes in average NLCD land use classes from 2001-2012

Land use in the study area changed slightly from 2001 to 2012. The average percentage for each class category was calculated for each subwatershed area for 2001 and 2012 (Table 3). Average land use change was calculated for the entire project area. The only significant change was in the “developed” category, which increased by 2.6 percent (figure 1).

##### Table 2.—Land use data for the Tuscaloosa stormwater assessment sites.

| Monitoring Site | Open Water % | Developed % | Forested % | Grassland/shrub/ scrub % | Agriculture % | Wetlands % |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 99.9 | 0 | 0 | 0 | 0.1 |
| 2 | 0 | 96.1 | 2.7 | 0.3 | 0.7 | 0.2 |
| 3 | 0 | 86.8 | 8.2 | 1.4 | 0.9 | 2.7 |
| 4 | 0.1 | 92.9 | 4.7 | 1.1 | 0.9 | 0.3 |
| 5 | 0 | 90.4 | 7.8 | 0.8 | 0.8 | 0.2 |
| 6 | 0.4 | 35.5 | 50 | 8.5 | 4 | 1.6 |
| 7 | 0 | 53.7 | 33.2 | 6.7 | 5.5 | 0.9 |
| 8 | 0.1 | 48.8 | 45.1 | 4 | 1.2 | 0.8 |
| 10 | 0 | 86.3 | 10 | 1 | 2.4 | 0.3 |
| 11 | 0 | 78.5 | 7.1 | 1.6 | 11.5 | 1.3 |
| 12 | 0.2 | 35.9 | 43 | 8.3 | 10.6 | 2 |
| 13 | 0.1 | 72.1 | 18.1 | 6.2 | 2.7 | 0.8 |
| 14 | 0.4 | 29.9 | 50.9 | 9.5 | 6.9 | 2.4 |
| 15 | 7 | 18.3 | 55.3 | 14.1 | 2.1 | 3.2 |
| 16 | 0.1 | 64.7 | 28 | 3.3 | 3.1 | 0.8 |
| 17 | 0.1 | 74.2 | 22.2 | 1.8 | 1 | 0.7 |
| 18 | 0.6 | 94.9 | 2.7 | 0.7 | 1 | 0.1 |
| 19 | 1.3 | 45.4 | 41.6 | 5.3 | 4.3 | 2.1 |
| 20 | 0 | 31.3 | 49.9 | 7.1 | 10.3 | 1.4 |

USEPA characterizes developed areas as having a high percentage (30 percent or greater) of constructed materials (i.e. asphalt, concrete, buildings, etc.). Developed areas were divided into four distinct categories: developed-open space, developed-low intensity, developed-medium intensity, and developed-high intensity. Developed-open space includes a mixture of some constructed materials, but mostly vegetation in the form of lawn grasses. Impervious surfaces account for less than 20 percent of this land-use category. These areas most commonly include large-lot single-family housing units, parks, golf courses, and vegetation planted in developed settings for recreation, erosion control, or aesthetic purposes. Developed-low intensity includes a mixture of constructed materials and vegetation. Impervious surfaces account for 20 to 49 percent of this category. These areas are composed of mostly single-family housing units. Developed-medium intensity includes areas with a mixture of constructed materials and vegetation. Impervious surfaces account for 50 to 79 percent of this category. These areas include mostly single-family housing units. Developed-high intensity includes highly developed areas where people reside or work in high numbers. Examples include apartment complexes, row houses, and commercial/industrial facilities. Impervious surfaces account for 80 to 100 percent of this category (table 3).

Table 3.—Developed areas for the Tuscaloosa stormwater assessment sites

| Monitoring Site | Developed open space % | Developed low intensity % | Developed medium intensity % | Developed high intensity % | Developed total % |
| --- | --- | --- | --- | --- | --- |
| 1 | 11 | 33.2 | 29.2 | 26.6 | 100 |
| 2 | 29.3 | 48.1 | 13.7 | 5 | 96.1 |
| 3 | 25.4 | 31.6 | 20.8 | 9 | 86.8 |
| 4 | 37.4 | 34.5 | 15.7 | 5.3 | 92.9 |
| 5 | 33.2 | 32.1 | 16.5 | 8.6 | 90.4 |
| 6 | 19.9 | 11.2 | 3.6 | 0.8 | 35.5 |
| 7 | 32 | 14.3 | 5.7 | 1.7 | 53.7 |
| 8 | 28.7 | 12.7 | 4.5 | 2.9 | 48.8 |
| 10 | 36.7 | 32.7 | 12.9 | 4 | 86.3 |
| 11 | 34.3 | 36 | 6.7 | 1.5 | 78.5 |
| 12 | 17.9 | 11.2 | 5.2 | 1.6 | 35.9 |
| 13 | 22.6 | 26.7 | 14.2 | 8.5 | 72 |
| 14 | 13 | 13.6 | 3 | 0.4 | 30 |
| 15 | 13.8 | 3.6 | 0.9 | 0 | 18.3 |
| 16 | 24.3 | 19.2 | 14.1 | 7 | 64.6 |
| 17 | 39.2 | 22.4 | 8.5 | 4.1 | 74.2 |
| 18 | 32.1 | 29.4 | 19.4 | 14 | 94.9 |
| 19 | 32.1 | 11.4 | 1.8 | 0.1 | 45.4 |
| 20 | 14.8 | 11.9 | 3.5 | 1.1 | 31.3 |

The impervious surface database was created by the MRLC consortium based upon the NLCD 2006 dataset; which is the same one used to derive the land-use/land-cover information. Impervious cover directly impacts water quality and quantity due to increased runoff. Watersheds can be classified by the percentage of impervious cover they are composed of. A classification system developed by T. Schueler in 1992 relates impervious cover to stream quality in three categories (table 4). The “sensitive” category represents 10% impervious cover and is potentially capable of supporting stable channels and good to excellent biodiversity (USEPA, 2002). The next category, “impacted”, represents 11% to 25% impervious cover that causes some impacts on stream quality. The last category, non-supporting, contains impervious cover greater than 25%. Streams in this category could benefit from restoration to reduce excessive erosion and siltation. Of the 19 monitoring sites in the project area, 11 are non-supporting, 4 are impacted, and 4 are in the sensitive category (fig. 2 and table 4).

Figure 2.—Impervious surfaces comparison for MRLC 2001 and 2006 data

The average percentage of impervious cover determined from the 2006 NLCD dataset for each monitored subwatershed was compared with the 2001 dataset. Of the 19 monitoring sites, 11 showed an increase in impervious cover, 7 showed a decrease, and 1 remained constant. The results of that comparison are shown in Table 4.

Table 4.—Impervious surface data for the Tuscaloosa stormwater assessment sites

| Monitoring Site | Sensitive (0-10%) | Impacted (11-25%) | Non-supporting (>25%) |
| --- | --- | --- | --- |
| 1 | 4.8 | 10.8 | 84.4 |
| 2 | 19.8 | 23.6 | 56.6 |
| 3 | 27.4 | 17.4 | 55.2 |
| 4 | 29.8 | 23.2 | 46.9 |
| 5 | 31.6 | 19.3 | 49.1 |
| 6 | 78.3 | 8.4 | 13.3 |
| 7 | 68.7 | 13.7 | 17.6 |
| 8 | 71.9 | 11.7 | 16.3 |
| 10 | 38.3 | 18.6 | 43.0 |
| 11 | 42.7 | 20.6 | 36.8 |
| 12 | 75.3 | 9.6 | 15.2 |
| 13 | 41.1 | 13.6 | 45.4 |
| 14 | 78.8 | 8.6 | 12.7 |
| 15 | 92.2 | 4.4 | 3.4 |
| 16 | 51.9 | 11.4 | 36.7 |
| 17 | 53.0 | 17.7 | 29.3 |
| 18 | 24.4 | 19.0 | 56.6 |
| 19 | 81.1 | 7.4 | 11.5 |
| 20 | 77.8 | 8.4 | 13.8 |

As previously mentioned, the most recent NLCD datasets used to calculate land-use/land-cover and impervious cover are derived from 2006 data. It is likely that both land use and impervious cover have changed since these datasets were created. It can be inferred that on average, the amount of development steadily increased from 2006 until present due to population growth and urban sprawl, with the exception of the 2011 tornado event. The tornado that devastated parts of Tuscaloosa on April 27, 2011 had a significant impact on both land use and its associated imperviousness along the tornado path. This reduction in urban development correlated with a decrease in imperviousness in the areas affected by the tornado. The transitioning phase that occurs after a natural disaster leaves the environment in a state of flux as natural processes adjust and communities rebuild.  Although the tornado was devastating, there may be opportunities for the city of Tuscaloosa to incorporate alternative development practices to reduce impervious surfaces and promote storm-water infiltration.

In the future, data will become available that can be analyzed using new methods to map and quantify impervious surfaces. One of these methods is a remote sensing approach that involves the classification of high spatial resolution imagery to create subpixel percent impervious surfaces that are mapped using a regression tree model. Another technique involves the NOAA ISAT (National Oceanic and Atmospheric Administration Impervious Surface Analysis Tool) Arcmap extension which is used to calculate the percentage of impervious surface area via remotely sensed land cover data. For the scope of this project, resources were not available to incorporate these methods of impervious cover estimation.

WATER QUALITY MONITORING

###### CHEMICAL AND PHYSICAL PARAMETERS

##### STREAM DISCHARGE

Discharge is a primary physical parameter that influences or affects surface-water quality in the project area. Ionic concentrations, specific conductance, dissolved oxygen (DO), biochemical oxygen demand (BOD), suspended and bedload sediment transport, and bacterial concentrations are all influenced by the volume and velocity of stream discharge.

The project subwatersheds have relatively small areal extents. However, the observed effects of urbanization, including flashy flow, large peak flow discharge, high flow velocities, and low base flow, are apparent when compared to streams in similar- sized watersheds with no urban development.

The effect of impervious surfaces on the volume of stormwater runoff can be dramatic. For example, 1 inch of rainfall on 1 acre of naturally vegetated land would typically produce 218 cubic feet of runoff. The same storm over a 1-acre paved parking lot would produce 3,450 cubic feet of runoff, nearly 16 times more than the natural landscape (Schueler, 1995).

Impacts associated with development typically go well beyond flooding and water quality.  The greater volume and intensity of runoff leads to increased erosion from construction sites, downstream areas, and stream banks.  Because a stream's shape evolves over time in response to the water and sediment loads that it receives, development-impacted runoff and sediment cause significant changes in stream form.

To facilitate increased flow, streams in urbanized areas tend to become deeper and straighter than wooded streams, and as they become clogged with eroded sediment, the ecologically important "pool and riffle" pattern of the streambed is usually destroyed (figure 3).

Figure 3.—Changes in stream form associated with urbanization.

These physical changes damage the ecological function of the stream.  Bank erosion and severe flooding destroy valuable streamside, or riparian, habitat.  Loss of tree cover leads to greater water temperature fluctuations, making the water warmer in the summer and colder in the winter.  Most importantly, loss of aquatic habitat is substantial, as the natural streambed composed of pebbles, rock ledges, riffles, and deep pools is covered by a uniform blanket of eroded sand and silt.

As urbanization increases, physical alterations including stream diversion, channelization, damming, and piping become common.  As these disturbances increase, so do the ecological impacts. The result may be a biologically sterile stream completely encased in underground concrete pipes.  In addition, related habitats like ponds and wetlands may be damaged or eliminated by grading and filling activities as green areas are converted to buildings, streets, and parking lots (fig. 4).

Figure 4.—Stylized relationship between percentage of imperviousness
 and receiving stream impact (adapted from Schueler, 1992).

STREAM TEMPERATURE

Water temperature is an important factor that affects the physical and geochemical characteristics of a stream. Dissolved oxygen, biological activity and equilibrium reactions are significantly influenced by water temperature. The standard for maximum temperature established by the Alabama Department of Environmental Management (ADEM, 1992) for surface water classified as Fish and Wildlife is 32.3º C. The standard was not exceeded in any monitored site during the current project period (table 5) or in the previous project period.

SPECIFIC CONDUCTANCE

Surface water in each project watershed is characterized by a unique specific conductance (SC) (microseimens/centimeter (µS/cm)) profile based on physical and chemical properties. The variability of SC is influenced by differences in stream temperature, discharge, total dissolved solids, local geology and soil conditions, and ionic influxes from nonpoint sources of pollution characteristic of urban runoff.  Streams without significant contaminant sources exhibit increased SC values with decreasing discharge due to increasing volumes of relatively high SC groundwater inflow and

| ##### Table 5.—Measured temperature values for Tuscaloosa stormwater assessment sites |  |  |  |
| --- | --- | --- | --- |
| Monitoring site | Maximum temperature (º C) | Minimum temperature (º C) | Average temperature (º C) |
| 1 | 21.0 | 12.4 | 16.5 |
| 2 | 20.0 | 11.4 | 15.1 |
| 3 | 20.0 | 11.3 | 15.1 |
| 4 | 20.0 | 14.0 | 15.5 |
| 5 | 19.0 | 11.0 | 14.2 |
| 6 | 19.0 | 11.5 | 14.5 |
| 7 | 18.0 | 12.6 | 14.7 |
| 8 | 17.0 | 13.1 | 15.4 |
| 9 | - | - | - |
| 10 | 21.0 | 11.5 | 15.4 |
| 11 | 16.9 | 11.3 | 14.4 |
| 12 | 15.5 | 12.9 | 13.8 |
| 13 | 15.8 | 12.0 | 13.3 |
| 14 | 15.0 | 11.6 | 13.2 |
| 15 | 15.0 | 11.0 | 13.2 |
| 16 | 15.2 | 11.0 | 12.9 |
| 17 | 16.4 | 11.0 | 13.3 |
| 18 | 17.2 | 10.7 | 14.5 |
| 19 | 16.2 | 11.0 | 13.0 |
| 20 | 15.2 | 12.0 | 13.5 |

decreasing SC with increasing discharge due to increasing volumes of relatively low SC runoff. The opposite SC character is exhibited for streams with significant contaminant sources where relatively high conductance runoff causes increasing SC with increasing discharge. Impacts of urban environments on SC are shown in table 6 where monitoring sites downstream from densely developed areas (sites 1, 3, 4, and 18) exhibited elevated SC. Figure 5 shows an SC profile for streams without significant nonpoint source contaminants.

pH

The concentration of hydrogen ions (H+) is a critical water quality parameter in natural and treated waters. Concentrations of hydrogen ions control speciation of other constituents, influence dissolution and precipitation of chemical elements, and determine whether the water will support aquatic life. Aquatic organisms are sensitive to pH change and require a pH of 5 to 9. Water treatment with specific types of chemicals, including disinfectants requires stringent pH control. Hydrogen ion activity, or pH, is controlled by interrelated chemical reactions that produce or consume hydrogen ions (Hem, 1985).

| Table 6.—Measured specific conductance values for the Tuscaloosa stormwater assessment sites. |  |  |  |
| --- | --- | --- | --- |
| Monitoring site | Maximum conductivity (µS/cm) | Minimum conductivity (µS/cm) | Average conductivity (µS/cm) |
| 1 | 140 | 14 | 60 |
| 2 | 129 | 29 | 71 |
| 3 | 332 | 138 | 214 |
| 4 | 143 | 32 | 88 |
| 5 | 92 | 28 | 61 |
| 6 | 77 | 30 | 58 |
| 7 | 97 | 47 | 79 |
| 8 | 96 | 40 | 56 |
| 9 | - | - | - |
| 10 | 105 | 24 | 65 |
| 11 | 98 | 60 | 79 |
| 12 | 95 | 32 | 68 |
| 13 | 88 | 28 | 61 |
| 14 | 76 | 31 | 56 |
| 15 | 72 | 52 | 61 |
| 16 | 97 | 50 | 73 |
| 17 | 113 | 51 | 68 |
| 18 | 130 | 66 | 87 |
| 19 | 48 | 12 | 39 |
| 20 | 93 | 55 | 73 |

Figure 5.—Measured specific conductance and discharge indicating no significant source of contamination in the watershed upstream from site 14.

Therefore, pH is an important indicator of the status of equilibrium reactions that determine the ionic composition of water that flows through the project watersheds. Highly variable pH observed in project streams (table 7) likely indicates influxes of contaminants related to urban runoff. In the 2005 project period, pH values were outside acceptable levels at three different sites on one occasion each. Levels of pH during the current sampling period all fell within the normal range.

##### Table 7.—Measured pH values for Tuscaloosa stormwater assessment sites.

| Monitoring Site | Maximum pH | Minimum pH | Average pH |
| --- | --- | --- | --- |
| 1 | 6.5 | 5.9 | 6.1 |
| 2 | 7.6 | 6.0 | 6.5 |
| 3 | 6.8 | 5.6 | 6.2 |
| 4 | 7.1 | 5.4 | 6.1 |
| 5 | 6.9 | 5.3 | 6.2 |
| 6 | 6.8 | 5.5 | 6.1 |
| 7 | 6.9 | 5.3 | 6.1 |
| 8 | 6.9 | 5.3 | 6.1 |
| 9 | - | - | - |
| 10 | 6.8 | 5.7 | 6.3 |
| 11 | 6.1 | 5.8 | 6.0 |
| 12 | 6.6 | 5.3 | 6.0 |
| 13 | 6.5 | 5.2 | 5.9 |
| 14 | 6.5 | 5.3 | 5.9 |
| 15 | 6.6 | 5.2 | 5.9 |
| 16 | 6.6 | 5.6 | 6.1 |
| 17 | 6.6 | 5.6 | 6.2 |
| 18 | 6.6 | 5.6 | 6.0 |
| 19 | 7.0 | 5.6 | 6.1 |
| 20 | 6.7 | 5.5 | 6.0 |

DISSOLVED OXYGEN

Dissolved oxygen (DO) concentration is critical to the biological health and the hydrochemical composition of surface waters. The ADEM standard for DO in surface water classified as Fish and Wildlife is 5.0 milligrams per liter (mg/L) except under extreme conditions, where it may be as low as 4.0 mg/L.

The equilibrium concentration of DO in water that is in contact with air is primarily related to water temperature and barometric pressure and secondarily related to concentrations of other solutes (Hem, 1985). Equilibrium DO in water at 10 ºC and 25ºC is 11.27 mg/L and 8.24 mg/L, respectively. DO concentrations in the project watersheds are significantly affected by water temperature, stream discharge, and concentrations of organic material in the water. However, the standard was not violated in the current project period (table 8) and was violated in only two samples in the previous project period.

##### Table 8.—Measured dissolved oxygen (DO) for Tuscaloosa

##### storm-water assessment sites 1-10.

| Monitoring sites | Dissolved Oxygen (mg/L) |
| --- | --- |
| 1 | 11.2 |
| 2 | 9.3 |
| 3 | 10.7 |
| 4 | 8.2 |
| 5 | 13.7 |
| 6 | 11.6 |
| 7 | 8.7 |
| 8 | 11.1 |
| 9 | - |
| 10 | 11.8 |

BIOCHEMICAL OXYGEN DEMAND

The biochemical oxygen demand (BOD) is an empirical measure of the amount of oxygen used for the biochemical oxidation of organic matter by the microbial population of a water body. This parameter may be used to indicate the presence and magnitude of organic pollutants. It is often used to determine the effect of waste discharges on the oxygen resources of receiving waters. BOD limitations for effluent established by the USEPA for biologically treated municipal wastewater is 30 mg/L. Standards established by some states for water-quality sensitive surface-water bodies may be as low as 5 mg/L (Mays, 1996). Observations from numerous streams monitored by the GSA throughout Alabama indicate that typical BOD values for streams with little or no urban influence vary from 0.5 to 4.0 mg/L. BOD values for both the previous project and current project (table 9) periods are typical of urban influenced streams. BOD values at monitored sites have generally increased from 2005 to 2013 (figure 6).

CHEMICAL OXYGEN DEMAND

The chemical oxygen demand (COD) test is a relatively quick method of determining pollution or oxidizable material load in surface water. The test yields an oxygen equivalent by the use of chemical oxidizer. A range of COD levels of <2 mg/L to

| Table 9.—Measured biochemical oxygen demand (BOD) for sites 1-10. |  |
| --- | --- |
| Monitoring sites | Biochemical oxygen demand (mg/L) |
| 1 | 8.0 |
| 2 | 7.0 |
| 3 | 4.0 |
| 4 | 7.0 |
| 5 | 7.5 |
| 6 | 6.9 |
| 7 | 7.9 |
| 8 | 5.3 |
| 9 | - |
| 10 | 5.4 |

Figure 6.—Comparison of 2005 and 2013 BOD concentrations.

100 mg/L generally is observed for natural streams and rivers (Mays, 1996). COD levels in area streams are in the upper reaches of values associated with natural rivers and streams (table 10). Compared with 2005 levels, 2013 levels are considerably lower with the exception of site 10 (figure 7).

TOTAL ORGANIC CARBON

Total organic carbon (TOC) analysis is a well-defined and commonly used methodology that measures the carbon content of dissolved and particulate organic matter

| Table 10.—Measured chemical oxygen demand (COD) for sites 1-10. |  |
| --- | --- |
| Monitoring sites | Chemical oxygen demand (mg/L) |
| 1 | 111 |
| 2 | 130 |
| 3 | 96 |
| 4 | 154 |
| 5 | 114 |
| 6 | 106 |
| 7 | 93 |
| 8 | 92 |
| 9 | - |
| 10 | 496 |

Figure 7.—Comparison of 2005 and 2013 COD concentrations.

present in water. Many water utilities monitor TOC to determine raw water quality or to evaluate the effectiveness of processes designed to remove organic carbon. Some wastewater utilities also employ TOC analysis to monitor the efficiency of the treatment process. In addition to these uses for TOC monitoring, measuring changes in TOC concentrations can be an effective "surrogate" for detecting contamination from organic compounds (e.g., petrochemicals, solvents, pesticides). Thus, while TOC analysis does not give specific information about the nature of the threat, identifying changes in TOC can be a good indicator of potential threats to a system (USEPA, 2005). Typical TOC values for natural waters vary from 1 to 10 mg/L (Mays, 1996). TOC values for monitoring sites 1-10 are shown in figure 8. All but two of the TOC values measured in the current project period are near or exceeding the 10mg/L threshold. Sites 6, 7, 8 and 10 have increased TOC values when compared with 2005 levels while sites 1, 2, 3 and 5 have decreased values. Site 4 has remained relatively static.

Figure 8.—Measured TOC concentrations at Sites 1-10.

TOTAL DISSOLVED SOLIDS

Natural water has suspended particles with a wide range of sizes that may, under favorable conditions, remain in suspension indefinitely. The smaller colloidal particles are considered to be in the dissolved state. Total dissolved solids (TDS) is a measure of these dissolved particles that remain in the water after larger suspended solids have been removed by filtering. TDS may be an indicator of water quality, especially in areas with land-use practices such as agriculture. In drinking water, an upper limit of 500 mg/L is desirable. The upper limit was not exceeded at any site in the current (table 11) or previous investigation.

| Table 11.—Measured total dissolved solids (TDS) for sites 1-20 |  |  |  |
| --- | --- | --- | --- |
| Monitoring site | Maximum TDS (mg/L) | Minimum TDS (mg/L) | Average TDS (mg/L) |
| 1 | 91 | 9 | 39 |
| 2 | 84 | 19 | 47 |
| 3 | 216 | 90 | 139 |
| 4 | 93 | 21 | 61 |
| 5 | 60 | 18 | 40 |
| 6 | 50 | 20 | 38 |
| 7 | 63 | 31 | 52 |
| 8 | 33 | 26 | 30 |
| 9 | - | - | - |
| 10 | 68 | 16 | 42 |
| 11 | 64 | 39 | 51 |
| 12 | 62 | 21 | 44 |
| 13 | 57 | 18 | 34 |
| 14 | 49 | 20 | 36 |
| 15 | 47 | 34 | 40 |
| 16 | 63 | 33 | 48 |
| 17 | 73 | 34 | 45 |
| 18 | 85 | 43 | 59 |
| 19 | 31 | 8 | 26 |
| 20 | 57 | 36 | 47 |

TURBIDITY

Turbidity in water is caused by suspended and colloidal matter such as clay, silt, finely divided organic and inorganic matter, and plankton and other microscopic organisms (Eaton, 1995). Turbidity is an expression of the optical property that causes light to be scattered and absorbed rather than transmitted with no change in direction or flux level through the stream (Eaton, 1995). Turbidity values measured from water samples may be utilized to formulate a rough estimate of long-term trends of total suspended solids (TSS). Turbidity data may also be used to evaluate the type of treatment necessary to remove sediment from water. However, turbidity data collected for the City of Tuscaloosa storm water assessment is used to compare monitored streams suspended sediment content (table 12). Typical turbidity values and corresponding stream discharge are given in figures 9 and 10. The highest average turbidity values were measured at sites 4, 5, 12, 13 and 19. The lowest values were measured at sites 7, 14, 16, and 17. Average turbidity in area streams is generally lower in the current study when compared with the previous study with the exception of sites 18, 19, and 20 (figure 11) and average discharge is higher with the exception of sites 2, 3, and 4 (figure 13).

| Table 12.—Measured turbidity in nephelometric turbidity units (NTU) for sites 1-20 |  |  |  |
| --- | --- | --- | --- |
| Monitoring site | Maximum turbidity Nephelometric Turbidity Units (NTU) | Minimum turbidity NTU | Average turbidity NTU |
| 1 | 129 | 33 | 59 |
| 2 | 220 | 35 | 104 |
| 3 | 71 | 19 | 36 |
| 4 | 955 | 14 | 217 |
| 5 | 989 | 27 | 279 |
| 6 | 277 | 56 | 162 |
| 7 | 398 | 7 | 177 |
| 8 | 279 | 28 | 148 |
| 9 | - | - | - |
| 10 | 284 | 42 | 128 |
| 11 | 409 | 30 | 141 |
| 12 | >1,000 | 18 | 302 |
| 13 | 989 | 10 | 224 |
| 14 | 417 | 9 | 139 |
| 15 | 417 | 10 | 93 |
| 16 | 234 | 7 | 128 |
| 17 | 194 | 3 | 79 |
| 18 | 182 | 108 | 142 |
| 19 | >1,000 | 10 | 238 |
| 20 | 481 | 47 | 223 |

Figure 9.—Measured turbidity and discharge for monitoring site 2.

Figure 10.—Measured turbidity and discharge for monitoring site 10.

Figure 11.—Comparison of 2005 turbidity values to 2013 values.

Figure 12.—Comparison of 2005 average discharge to 2013 average discharge.

CONSTITUENT CONCENTRATIONS AND LOADING IN PROJECT STREAMS

NUTRIENTS IN PROJECT STREAMS

A typical aquatic ecosystem includes plants and animals composed of carbon, hydrogen, oxygen, nitrogen, phosphorus, and sulfur. These carbonaceous, nitrogenous, phosphorous, and sulfurous substances decompose upon death of plants and animals and serve as nutrients for development of new organisms. However, excessive concentrations of these substances in the aquatic environment may lead to increased biological activity, decreased dissolved oxygen concentrations, and decreased numbers of species (Mays, 1996). This process is called eutrophication. Concentrations of nitrogen and phosphorus are of primary importance for the health of surface-water bodies. Organic matter contains organic nitrogen, which degrades to ammonia in the first step of the nitrification process. Ammonia is then oxidized to form nitrite. Continued oxidation of nitrite forms nitrate. The largest concentrations of ammonia, nitrate, and phosphorus were measured at sites 1, 7 and 10 respectively. This indicates the presence of one or more sources of water contamination in these watersheds.

AMMONIA

Concentrations of ammonia (NH3-N) in uncontaminated streams may be as low as 0.01 mg/L. Concentrations of ammonia in contaminated streams and in streams downstream from wastewater discharges are generally from 0.5 to 3.0 mg/L.

Concentrations higher than 0.5 mg/L may cause significant ammonia toxicity to fish and other organisms (Maidment, 1993). The toxicity limit (0.5 mg/L) was not exceeded in the current project period (Table 13). Site 2 exceeded the toxicity limit in the 2005 project period.

##### Table 13.—Measured concentrations of ammonia (NH3 as N) for sites 1-10

| Monitoring site | Ammonia (NH3 as N) (mg/L) |
| --- | --- |
| 1 | 0.270 |
| 2 | 0 |
| 3 | 0.093 |
| 4 | 0.150 |
| 5 | 0.103 |
| 6 | 0.090 |
| 7 | 0.073 |
| 8 | 0 |
| 9 | - |
| 10 | 0.202 |

##### NITRATE

The USEPA maximum contaminant level (MCL) for nitrate in drinking water is 10 mg/L. Typical nitrate (NO3 as N) concentrations in streams vary from 0.5 to 3.0 mg/L. Concentrations of nitrate in streams without significant nonpoint sources of pollution vary from 0.1 to 0.5 mg/L. Streams fed by shallow ground water draining agricultural areas may approach 10 mg/L (Maidment, 1993). The nitrate standard for streams without significant nonpoint sources of pollution (0.5 mg/L) was exceeded by samples collected at Sites 5, 6, 7,  and 8 (table 14, fig.12, pl. 3) in the current project period. All samples collected exceeded the standard in the 2005 study period.

##### Table 14.—Measured concentrations of nitrate (NO3 as N) for sites 1-10

| Monitoring site | Nitrate (NO3 as N) (mg/L) |
| --- | --- |
| 1 | .447 |
| 2 | .180 |
| 3 | .144 |
| 4 | .343 |
| 5 | .574 |
| 6 | .528 |
| 7 | .881 |
| 8 | .711 |
| 9 | - |
| 10 | .433 |

Figure 12.—Measured Nitrate concentrations at sites 1-10.

PHOSPHORUS

Natural background concentration of total dissolved phosphorus is approximately 0.025 mg/L (Maidment, 1993). Phosphorus concentrations as low as 0.01 to 0.005 mg/L may cause excessive algae growth, but the critical level of phosphorus necessary for excessive algae is around 0.05 mg/L (Maidment, 1993). In many streams phosphorus is the primary nutrient that influences excessive biological activity (Maidment, 1993).

##### Phosphorus concentrations in monitored streams were elevated during the current sampling period (table 15). Concentrations of phosphorus necessary for excessive algae growth were measured at all sites. Site 10 exhibited the largest concentrations of total phosphorus (table 15, fig. 13, pl. 4). Sites 1, 2 and 7-10 exceeded the concentration necessary for algae growth in the 2005 study. Most concentrations of phosphorus in natural and waste waters occur as phosphates. These phosphates arise from a variety of sources such as laundering or cleaning solutions and agricultural or residential fertilizers. Organic phosphates are formed primarily by biological processes and they contribute to sewage by body waste and food residues (Eaton and others, 1995).

| Table 15.—Measured concentrations of total phosphorus for sites 1-10 |  |
| --- | --- |
| Monitoring site | Total phosphorus (mg/L) |
| 1 | .144 |
| 2 | .315 |
| 3 | .098 |
| 4 | .236 |
| 5 | .166 |
| 6 | .140 |
| 7 | .103 |
| 8 | .133 |
| 9 | - |
| 10 | .684 |

Figure 13.—Measured Phosphorus concentrations at sites 1-10.

##### BACTERIA

Microorganisms are present in all surface waters and include viruses, bacteria, fungi, algae, and protozoa. Analyses of bacteria levels may be used to assess the quality of water and to indicate the presence of human and animal waste in surface and ground water. Fecal coliform and fecal streptococcus groups of bacteria are used as the primary indicator organisms of this type of water pollution. The membrane filter procedure as described in the 19th Edition of Standard Methods for the Examination of Water and Wastewater (Eaton and others, 1995) was used for determining fecal coliform bacteria counts for water samples.

The flushing action of stormwater runoff causes increased concentrations of nonpoint-source pollutants in receiving streams. Previous studies performed by the GSA have demonstrated excellent correlations between increased stream discharge and increased concentrations of in-stream bacteria. Possible sources of fecal contamination to surface waters include wastewater treatment plants, on-site septic systems, domestic and wild animal manure, and storm runoff (USEPA, 2004).

The limit for fecal coliform bacteria, established for surface waters classified as Fish and Wildlife is 2,000 colonies per 100 milliliter sample for single samples (ADEM, 1992). Sampling results indicate that the single limit was exceeded at all collected sites during the current study period (table 16, fig.14) and the previous study period. The fecal coliform counts determined for project streams are extremely large but are not uncommon from streams in urban environments. The high counts at sites 2 and 10 combined with high levels of phosphorus indicate the possibility of sewer leakage or some other large contaminant source.

METALLIC CONSTITUENTS

Water samples collected from monitoring sites 1-10 were analyzed for selected metallic constituents. Results are similar to those observed in the 2005 study. The USEPA compiled national recommended water quality criteria for the protection of aquatic life and human health in surface water for approximately 150 pollutants. These criteria are published pursuant to Section 304(a) of the Clean Water Act (CWA) and provide guidance for states and tribes to use in adopting water quality standards (USEPA,

| Table 16.—Measured concentrations of fecal coliform for sites 1-10. |  |
| --- | --- |
| Monitoring site | Fecal coliform (col/100ml) |
| 1 | 155,310 |
| 2 | 464,000 |
| 3 | 43,250 |
| 4 | 241,960 |
| 5 | 141,360 |
| 6 | 104,620 |
| 7 | 98,040 |
| 8 | 173,290 |
| 9 | - |
| 10 | 4,106,000 |

Figure 14.—Measured fecal coliform bacteria concentrations for sites 1-10.

2009). The criteria were developed for acute (short-term exposure) and chronic (long-term exposure) concentrations. Tables 17 and 18 show measured metals and their recommended acute and chronic maximum concentrations. Many constituents observed in surface water are naturally occurring and originate from the dissolution or erosion of

Table 17.—Measured concentrations of metallic constituents detected in water samples for Tuscaloosa stormwater assessment sites 1-5.

| Metallic constituent (µg/L) | Monitoring site 1 | Monitoring site 2 | Monitoring site 3 | Monitoring site 4 | Monitoring site 5 | USEPA standards for protection of aquatic life (µg/La) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Acute | Chronic |
| Aluminum | BDL | 83 | BDL | 95 | BDL | 750 | 87 |
| Antimony | BDL | BDL | BDL | BDL | BDL | N/A | N/A |
| Arsenic | 1.12 | 1.09 | 1.72 | 1.21 | 0.68 | 340 | 150 |
| Barium | 27.2 | 37.1 | 165.5 | 57.8 | 57.3 | N/A | N/A |
| Beryllium | BDL | BDL | BDL | BDL | BDL | N/A | N/A |
| Cadmium | BDL | BDL | BDL | BDL | BDL | 2 | 0.25 |
| Calcium | 6,880 | 8,410 | 43,200 | 9,520 | 9,110 | N/A | N/A |
| Chromium | BDL | 0.57 | BDL | 0.64 | 0.61 | 570 | 74 |
| Copper | BDL | BDL | BDL | BDL | BDL | 4.67 | N/A |
| Iron | 38.9 | 43.0 | 332.0 | 109.0 | 137.0 | N/A | 1,000 |
| Lead | 0.71 | 0.73 | 0.63 | 0.71 | 0.56 | 65 | 2.5 |
| Lithium | BDL | BDL | BDL | BDL | BDL | N/A | N/A |
| Magnesium | 656.0 | 697.0 | 5,170 | 1,110 | 1,210 | N/A | N/A |
| Manganese | 18.3 | 18.5 | 419.0 | 42.7 | 51.9 | N/A | N/A |
| Mercury | BDL | 0.028 | 0.031 | 0.037 | BDL | 1.4 | 0.77 |
| Molybdenum | BDL | BDL | BDL | BDL | BDL | N/A | N/A |
| Nickel | BDL | BDL | BDL | BDL | BDL | 470 | 52 |
| Potassium | 2,540 | 2,530 | 5,760 | 2,380 | 1,830 | N/A | N/A |
| Selenium | BDL | BDL | BDL | BDL | BDL | N/A | 5.0 |
| Silver | BDL | BDL | BDL | BDL | BDL | 3.2 | N/A |
| Sodium | 1.00 | .94 | 10.2 | 1.47 | 1.40 | N/A | N/A |
| Strontium | .0138 | .0171 | .1630 | .0171 | .0155 | N/A | N/A |
| Thallium | BDL | BDL | BDL | BDL | BDL | N/A | N/A |
| Tin | BDL | BDL | BDL | BDL | BDL | N/A | N/A |
| Vanadium | BDL | BDL | BDL | BDL | BDL | N/A | N/A |
| Zinc | .0399 | .0520 | .0524 | .1090 | .0752 | 120 | 120 |

rocks or sediments. In most cases, these constituents occur in relatively small concentrations and have no detrimental effects on the environment or human health. Barium, calcium, potassium, sodium, and strontium are very common in many rock types (Hem, 1985) and are common in aquatic environments. All samples collected during the project contained these constituents in relatively small concentrations.

Table 18.—Measured concentrations of metallic constituents detected in water samples for Tuscaloosa stormwater assessment sites 6, 7, 8, and 10.

| Metallic constituent (µg/L) | Monitoring site 6 | Monitoring site 7 | Monitoring site 8 | Monitoring site 10 | USEPA standards for protection of aquatic life (µg/La) |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Acute | Chronic |
| Aluminum | 64.0 | 86.0 | 132.0 | BDL | 750 | 87 |
| Antimony | BDL | BDL | BDL | BDL | N/A | N/A |
| Arsenic | 0.73 | 0.62 | 0.62 | 0.86 | 340 | 150 |
| Barium | 72.7 | 74.8 | 71.1 | 75.2 | N/A | N/A |
| Beryllium | BDL | BDL | BDL | BDL | N/A | N/A |
| Cadmium | BDL | BDL | 0.1 | BDL | 2 | 0.25 |
| Calcium | 11,300 | 15,800 | 6,380 | 11,800 | N/A | N/A |
| Chromium | BDL | BDL | 12.20 | 0.32 | 570 | 74 |
| Copper | BDL | BDL | BDL | BDL | 4.67 | N/A |
| Iron | 211.0 | 311.0 | 356.0 | 204.0 | N/A | 1,000 |
| Lead | 0.65 | 1.20 | 2.76 | 0.86 | 65 | 2.5 |
| Lithium | BDL | BDL | BDL | BDL | N/A | N/A |
| Magnesium | 2,450. | 3,730. | 1,500. | 2,290 | N/A | N/A |
| Manganese | 228.0 | 171.0 | 087.8 | 122.0 | N/A | N/A |
| Mercury | BDL | 0.264 | 0.0051 | BDL | 1.4 | 0.77 |
| Molybdenum | BDL | BDL | BDL | BDL | N/A | N/A |
| Nickel | BDL | BDL | BDL | BDL | 470 | 52 |
| Potassium | 2,390 | 2,540 | 2,350 | 2,970 | N/A | N/A |
| Selenium | BDL | BDL | BDL | BDL | N/A | 5.0 |
| Silver | BDL | BDL | BDL | BDL | 3.2 | N/A |
| Sodium | 2,370 | 3,130 | 2,160 | 3,670 | N/A | N/A |
| Strontium | 21.4 | 32.7 | 19.3 | 20.2 | N/A | N/A |
| Thallium | BDL | BDL | BDL | BDL | N/A | N/A |
| Tin | BDL | BDL | BDL | BDL | N/A | N/A |
| Vanadium | BDL | BDL | BDL | BDL | N/A | N/A |
| Zinc | 66.4 | 89.9 | 204.0 | 81.4 | 120 | 120 |

*BDL = Below detectable limit.

Arsenic, nickel, and zinc also occur naturally in metamorphic and igneous rocks and are present in some waters (Hem, 1985) but are much less common than barium, calcium, potassium, sodium, and strontium. Arsenic is used in pesticides and industrial processes. Nickel and zinc are used in metallurgical processes, and zinc is used as a white pigment in paint and rubber. Detections of arsenic and nickel were all below the USEPA criteria (tables 17, 18). Arsenic and zinc were detected at all sites while nickel was not detected. Zinc was measured at a concentration exceeding the USEPA criteria at site 8 (table 18).

Lead, usually in small concentrations, is pervasive in the environment. The sources of lead are varied and include industrial waste and atmospheric transport from regional or intercontinental sources. Lead was detected in all sampled watersheds but only exceeded the USEPA criteria at site 8. The source of lead in this watershed is unknown.

Shale in the Pottsville Formation and clay in the Tuscaloosa Group have large, naturally occurring concentrations of iron, aluminum, and manganese. All monitoring sites with high concentrations of these constituents have significant amounts of shale and clay exposed and disturbed due to excavation associated with construction activities or eroded from stream banks and beds. It is probable that high concentrations of these constituents are related to construction activities and erosion in the watersheds.

INORGANIC NONMETALLIC CONSTITUENTS

Water samples were analyzed for selected inorganic nonmetallic constituents. Tables 19 and 20 contain the concentrations of inorganic nonmetallic constituents. Results are similar to those observed in the 2005 study. Although no drinking water standard currently exists for Boron, concentrations as small as 1 mg/L may be toxic to plant life (Hem, 1985). Boron is occasionally detected in the surface waters of Alabama. Boron is naturally associated with igneous rocks and is present in active volcanic areas. In areas without a natural source, it may originate from cleaning wastes and may be present in sewage and industrial wastes (Hem, 1985). Boron was detected at sites 3, 4, 6 and 7, although concentrations were relatively small (tables 19, 20).

Chloride, silica, and sulfate were detected at all monitored sites and fluoride was detected at sites 1, 3, 4, and 7. These constituents are common in surface water and usually originate, in the observed range of concentrations, from sediments that underlie the monitored watersheds.

Table 19.—Measured concentrations of inorganic nonmetallic constituents detected for the Tuscaloosa stormwater assessment sites 1-5

| Inorganic nonmetallic constituent (mg/L) | Monitoring site 1 | Monitoring site 2 | Monitoring site 3 | Monitoring site 4 | Monitoring sites 5 |
| --- | --- | --- | --- | --- | --- |
| Boron | BDL | BDL | .197 | .114 | BDL |
| Bromide | BDL | BDL | .24 | BDL | BDL |
| Cyanide | BDL | BDL | .0046 | BDL | BDL |
| Chloride | 2.83 | .85 | 6.72 | 2.04 | 2.38 |
| Fluoride | .074 | BDL | .074 | .069 | BDL |
| Silica | .88 | 1.20 | 7.15 | 1.56 | 1.77 |
| Sulfate | 3.55 | 1.88 | 2.78 | 4.23 | 2.48 |

Table 20.—Measured concentrations of inorganic nonmetallic constituents detected for the Tuscaloosa stormwater assessment sites 6-10

| Inorganic nonmetallic constituent (mg/L) | Monitoring site 6 | Monitoring site 7 | Monitoring site 8 | Monitoring site 9 | Monitoring sites 10 |
| --- | --- | --- | --- | --- | --- |
| Boron | .063 | .051 | BDL | - | BDL |
| Bromide | BDL | BDL | BDL | - | BDL |
| Cyanide | BDL | BDL | BDL | - | BDL |
| Chloride | 5.00 | 5.08 | 3.27 | - | 4.34 |
| Fluoride | BDL | .092 | BDL | - | BDL |
| Silica | 4.01 | 1.00 | 3.97 | - | 3.06 |
| Sulfate | 4.20 | 7.61 | 2.85 | - | 4.64 |

ORGANIC CONSTITUENTS

Organic compounds are commonly used in our society today. Frequently, these compounds appear in streams and ground-water aquifers. Many of these compounds are harmful to human health and to the health of the aquatic environment. A limited number of organic constituents were analyzed in water samples collected from the monitored sites (comprehensive analytical monitoring sites). They include TOC (discussed previously) phenol, and oil and grease. Concentrations observed are similar to those observed in the 2005 study.

Phenols are used in the production of phenolic resins, germicides, herbicides, fungicides, pharmaceuticals, dyes, plastics, and explosives (USGS, 1992-96). They may occur in domestic and industrial wastewaters, natural waters, and potable water supplies. They generally are traceable to industrial effluents or landfills (Eaton and others, 1995). The EPA states that phenols should be limited to 0.3 ug/L (micrograms per liter) in lakes and streams to protect humans from the possible harmful effects of exposure. Phenols cause acute and chronic toxicity to freshwater aquatic life. Phenol concentrations were well above recommended levels in all monitored streams. This indicates the ubiquitous nature of this compound in urban environments (table 21, fig.15).

Table 21.—Measured concentrations of phenol for sites 1-10

| Monitoring site | Phenol ug/L |
| --- | --- |
| 1 | 10.3 |
| 2 | 10.9 |
| 3 | 4.2 |
| 4 | 10.8 |
| 5 | 7.1 |
| 6 | 12.3 |
| 7 | 10.6 |
| 8 | 6.0 |
| 9 | - |
| 10 | 29.8 |

Figure 17.—Measured phenol concentrations for sites 1-10.

Concentrations of oil and grease were determined for water samples collected from sites 1-10. Oil and grease includes fatty matter from animal and vegetable sources and from hydrocarbons of petroleum origin. Concentrations were not detected above the detection limit of 5.0 mg/L (table 22).

Table 22.—Measured concentrations of oil and grease for sites 1-10

| Monitoring site | Oil and grease mg/L |
| --- | --- |
| 1 | BDL |
| 2 | BDL |
| 3 | BDL |
| 4 | BDL |
| 5 | BDL |
| 6 | BDL |
| 7 | BDL |
| 8 | BDL |
| 9 | - |
| 10 | BDL |

- Volatiles, semi-volatiles, herbicides, pesticides, and PCBs were analyzed for each water sample collected at the comprehensive monitoring sites. None of these compounds were detected in the samples. These analytical results are included in the appendix.

SEDIMENTATION

Sedimentation is a process by which eroded particles of rock are transported primarily by moving water from areas of relatively high elevation to areas of relatively low elevation, where the particles are deposited. Upland sediment transport is primarily accomplished by overland flow and rill and gully development. Lowland or floodplain transport occurs in streams of varying order, where upland sediment joins sediment eroded from floodplains, stream banks, and streambeds. Erosion rates are accelerated by human activity related to agriculture, construction and urban development, timber harvesting, unimproved roadways, or any activity where soils or geologic units are exposed or disturbed. Excessive sedimentation is detrimental to water quality, destroys biological habitat, reduces storage volume of water impoundments, impedes the usability of aquatic recreational areas, and causes damage to structures. Sediment loads in streams are primarily composed of relatively small particles suspended in the water column (suspended solids) and larger particles that move on or periodically near the streambed (bedload).

Twenty monitoring sites were established in 2004 throughout the City of Tuscaloosa to measure sediment loads, flow conditions, and field water-quality parameters. Nineteen of those sites were accessible and were monitored during 2012 and 2013. Comparisons of 2004-2005 sediment transport rates are made with the 2012-2013 data in the following text.

Sediment Loads Transported by Project Streams

The rate of sediment transport is a complex process controlled by a number of factors including land use, precipitation runoff, erosion, stream discharge and flow velocity, stream base level, and physical properties of the stream and sediment. All of these factors influence sediment transport rates observed in the Tuscaloosa streams. Most urban areas are defined by constant, large-scale human activity involving modification of the natural environment including streams (fig. 18). The City of Tuscaloosa is no exception. At any time, numerous construction projects are ongoing in the city that involve removal of natural vegetation, disturbance of soils, and construction of

Stream bank erosion

Bed sediment

Site 13

Figure 18.—Stream conditions downstream from site 13.

impervious surfaces. All of these factors contribute to the sediment loads transported by streams. Additional disturbances were caused by the April 27, 2011 tornado, which destroyed a significant part of the city and left large areas void of vegetative cover.

Sediment is supplied to streams from two primary sources. The first is from erosion of natural geologic materials that form stream channels and banks and form soils in the project area. The second is from fill material transported to construction sites in the city. Due to the nature of available fill material and the character of bedload observed in project streams, it is assumed that this material is obtained locally. The Fall Line (updip limit of unconsolidated coastal plain sediments) bisects the City of Tuscaloosa (fig. 18). The eastern and northern portions of the city are underlain by sandstone and shale of the upper part of the Pottsville Formation. The Pottsville may be observed in valley floors and in stream channels and floodplains. The ridges in this area consist of sand, gravel, and clay of the high terrace deposits associated with the ancestral Black Warrior River and of the Coker Formation. The southern and western areas of the city are underlain by sand, gravel, and clay of alluvial, coastal, and low terrace deposits and by the Coker Formation (Osborne and others, 1988). The Pottsville and Coker Formations are relatively resistant to erosion. However, the increased volume and velocity of runoff related to urban storm water drainage erodes these materials and causes them to be significant sources of sediment.

Excessive sedimentation causes changes in base level elevation of streams and triggers downstream movement of the material as streams attempt to regain base level equilibrium. The movement of this material is accelerated by periodic extreme precipitation events that cause increased stream flow and stream flow velocity.

Due to the grain size distribution of sediment in Tuscaloosa watersheds, movement of the material is controlled primarily by stream gradients and resulting stream flow velocities. Large amounts of clay and silt may be suspended in the water column and transported at relatively low velocities in some streams during any discharge event greater than base flow. However, much of the bedload material in the project area consists of coarse sand and gravel eroded from unconsolidated sediments and silt, sand, and cobbles eroded from geologic units in the area.

Figure 19.—Geologic map of Tuscaloosa County showing Fall Line

(modified from GSA, 2006).

In order for the bedload material to be transported, a critical flow velocity threshold must be exceeded. This occurs during significant precipitation events. The duration of each pulse of bedload migration is dependent on the magnitude and duration of the discharge event. Once the streambed material is mobilized, the level of energy required to keep the material moving on the falling limb of the hydrograph is much less. Therefore, large amounts of bedload will continue to be transported even as streams approach base flow conditions. Examples of critical velocities for bedload movement in project streams are shown in figures 20 and 21.

Figure 20.—Critical stream flow velocity determined from measured velocities at site 12, Rum Creek at Highway 69.

Figure 21.—Critical stream flow velocity determined from measured velocities at site 5,

Cypress Creek at Highway 69.

###### Stream Flow Conditions

Sediment transport conditions in watersheds in the City of Tuscaloosa are segregated by instream conditions, urban development, and current land use activities. Instream conditions in this urban environment include channel width, stream bank shape and composition, streambed composition, and stream gradient. Stream flow within the City of Tuscaloosa is profoundly impacted by modifications to stream channels to accommodate urban development and to accelerate movement of storm water runoff out of the city. All monitored streams were modified by some type of channelization including concrete flume, limestone boulder armoring, or natural geologic material. Individual streams may include two or possibly all three stream channel types.

Urban discharge is characterized by extremely high and low flows when compared to streams in rural areas. Most precipitation events cause rapid increases in discharge but most events are of relatively short duration.

Monitoring periods were chosen to collect samples and instream data from well-distributed discharge events varying from base flow to flood. Each stream was monitored sufficiently to afford comparisons with 2004-2005 and 2012-2013 data.

Suspended Sediment

The basic concept of constituent loads in a river or stream is simple. However, the mathematics of determining a constituent load may be quite complex. The constituent load is the mass or weight of a constituent that passes a cross-section of a stream in a specific amount of time. Loads are expressed in mass units (e.g., tons, kilograms) and are considered for time intervals that are relative to the type of pollutant and the watershed area for which the loads are calculated. Loads are calculated from concentrations of constituents obtained from analyses of water samples and stream discharge, which is the volume of water that passes a cross-section of the river in a specific amount of time.

The computer model Regr_Cntr.xls (Regression with Centering) was selected to calculate suspended sediment loads for this project. The program is an Excel adaptation of the USGS seven-parameter regression model for load estimation (Cohn and others, 1992). It estimates loads in a manner very similar to that used most often by the Estimatr.exe (USGS Estimator) program. The Regr_Cntr.xls program was adapted by R. Peter Richards at the Water Quality Laboratory at Heidelberg College (Richards, 1999). The program establishes a regression model using a calibration set of data composed of concentrations of the constituent of interest and discharge values measured at the time of water sampling. These data are applied to mean annual daily discharge to obtain mean daily loads. The mean daily values are summed to obtain an annual load. Constituent loads can be estimated for any year for which mean daily discharge data are provided. Mean daily discharge was determined by calculating a ratio between measured discharge for each monitored site and discharge for the same time period obtained from the USGS real time discharge gauge located on Cribbs Mill Creek near Kauloosa Avenue. The ratio established for each monitored site was then applied to mean daily discharge from the USGS gauge to obtain mean daily discharge for each site. Loads from the 2004-2005 project were compared to those monitored in 2012 and 2013 by using the 2004-2005 mean daily discharge values with sediment volumes measured for both time periods.

Suspended sediment is defined as that portion of a water sample that is separated from the water by filtering. This solid material may be composed of organic and inorganic material that includes algae, industrial and municipal wastes, urban and agricultural runoff, and eroded material from geologic formations. These materials are transported to stream channels by overland flow related to storm-water runoff.

Suspended sediment typically composes more than 50 percent of the total sediment loads. In urban environments where stream banks and beds may be armored with concrete or limestone boulders almost all sediment is suspended during storm events when flow velocities are elevated. Total sediment loads determined for nine of twenty project sites were assumed to be composed of suspended sediment. Annual suspended sediment loads are given as total load in tons per year (t/yr) and normalized load with respect to unit area (t/mi2/yr). Estimated sediment load data represent land uses occurring in the watersheds at the time of sampling.

The concentrations of suspended sediment in mg/L were determined by laboratory analysis of individual samples collected at each monitoring site over one year at variable stream discharge rates. Suspended sediment loads for each stream during the monitoring period were determined using measured total suspended solids (TSS) concentrations, and estimated mean daily discharge values were entered into the regression model. The relationship between discharge and TSS may be graphically represented by a regression curve that may be used to describe the movement of suspended sediment through a stream system.

Sediment loads are constantly changing with changes in land use and climate. Comparisons of 2004-2005 and 2012-2013 data indicates that suspended sediment loads increased at two sites, decreased at 11 sites, and remained relatively constant at five sites. In each case, comparisons can be explained by observations of land use and climate. Figure 21 shows the comparison of TSS concentrations at site 4 (Cribbs Mill Creek and Kauloosa Avenue), which had the largest increase from the 2004-2005 to

Figure 21.—TSS concentrations for samples collected at site 4 during the 2004-2005

and 2012-2013 monitoring periods.

the 2012-2013 monitoring period. This increase is most likely the result of land disturbance related to the April 27, 2011 tornado, since Cribbs Mill Creek flows through the city in the tornado path (fig. 22).

The largest suspended load estimated during 2004-2005 monitoring occurred at site 13 (Cypress Creek at U.S. Highway 82) (18,932 t/yr). A large amount of constructionoccurred upstream from this site during the monitoring period. The suspended load for the 2012-2013 period was 217 t/yr, indicating that many of the disturbed areas are now stabilized (fig. 23). The second largest load during 2004-2005 was estimated at site 6 (tributary to Cottondale Creek at JVC Road) (15,479 t/yr). Large amounts of residential and commercial construction, including a large excavation project, was ongoing upstream from this site. This load increased slightly to 16,500 t/yr during the 2012-2013 monitoring period, indicating that there continue to be significant sources of

Cribbs Mill Creek

drainage area

Figure 22.—April 27, 2011 Tuscaloosa tornado path and Cribbs Mill Creek drainage area (modified from city of Tuscaloosa, 2011).

sediment in this watershed. The smallest load estimated during 2004-2005 was at site 1 (unnamed tributary to the Black Warrior River at Jack Warner Parkway near the U.S. Army Corps of Engineers facility) (60 t/yr). This load decreased to16 t/yr in 2012-2013. However, sites 10 (unnamed tributary to the Black Warrior River near Indian Hills Country Club) and 11 (unnamed tributary to Moody Swamp at Fosters Ferry Road) had the smallest loads (8 and 7 t/yr, respectively). Loading at site 11 could not be estimated for the 2004-2005 period due to multiple TSS regressions (fig. 24). Periodically during the 2004-2005 monitoring period, TSS in this stream increased significantly although the stream remains at or near base flow. Conversely, at times of increased discharge, TSS

−

Figure 23.— TSS concentrations for samples collected at site 13 during the 2004-2005

and 2012-2013 monitoring periods.

concentrations remained relatively small (less than 25 mg/L). Most likely, the suspended material was not sediment,but was the result of upstream dumping of some unknown contaminant in the stream that was concentrated during low flow and diluted during increased flow conditions. Apparently, the upstream dumping was discontinued so that the stream now has a normal flow/suspended sediment relationship (fig. 24).

Site 1 includes the downtown area and has the largest amount of impervious surface and the smallest amount of construction and erosion. Sites 10 and 11 are established residential and commercial areas with relatively little disturbance. Estimated annual suspended sediment loads for the 2004-2005 and 2012-2013 monitoring periods are shown in figure 25.

Figure 24.—Multiple TSS/discharge regressions for 2004-2005 monitoring period and single regression for the 2012-2013 monitoring period.

Normalization of the 2004-2005 suspended sediment load data with respect to unit watershed area indicated that Cypress Creek sites 13 and 14 had the largest suspended loads (5,737 and 4,945 t/mi2/yr, respectively). Site 3 had the smallest load (56 t/mi2/yr). Normalized 2012-2013 data shows that sites 4 (Cribbs Mill Creek and Kauloosa Avenue) and site 6 (tributary to Cottondale Creek at JVC Road) have the largest loads (4,295 and  3,618 t/mi2/yr, respectively). Site 10 (unnamed tributary to the Black Warrior River near Indian Hills Country Club) has the smallest normalized load (8 t/mi2/yr). Estimated annual normalized suspended sediment loads for the 2004-2005 and 2012-2013 monitoring periods are shown in figure 26.

Figure 25.—Estimated suspended sediment loads for city of Tuscaloosa monitored watersheds for the 2004-2005 and 2012-2013 monitoring periods.

Bed Sediment

Transport of streambed material is controlled by a number of factors primarily related to stream discharge and flow velocity, erosion and sediment supply, stream base level, and physical properties of the streambed material. Most streambeds are in a state of constant flux in order to maintain a stable base level elevation. The energy of flowing water in a stream is constantly changing to supply the required power for erosion or deposition of bed sediment to maintain equilibrium with the local water table and regional or global sea level. Stream base level may be affected by regional or global events including fluctuations of sea level or tectonic movement. Local factors affecting base level include fluctuations in the water table elevation, changes in the supply of

Figure 26.-- Estimated normalized suspended sediment loads for city of Tuscaloosa monitored watersheds for the 2004-2005 and 2012-2013 monitoring periods.

sediment to the stream caused by changing precipitation rates, and /or land use practices that promote excessive erosion in the floodplain or upland areas of the watershed. Bed sediment is composed of particles that are too large or too dense to be carried in suspension by stream flow.

Figure __.—Estimated suspended sediment loads for city of Tuscaloosa monitored watersheds during the 2004-2005 and 2012-2013 monitoring periods

These particles roll, tumble, or are periodically suspended as they move downstream. Traditionally, bed sediment has been difficult to quantify due to deficiencies in monitoring methodology or inaccuracies of estimating volumes of sediment being transported along the streambed. This is particularly true in streams that flow at high velocity or in streams with excessive sediment loads.

Staff of the Geological Survey of Alabama developed a portable bed sedimentation rate-monitoring device designed to accurately measure bed sediment in shallow sand or gravel bed streams. This device was utilized in the City of Tuscaloosa project watersheds for the 2004-2005 monitoring period and again during 2012-2013, where bed sediment was measured periodically during the project period to obtain a well-distributed dataset with respect to stream discharge and velocity. These data were used to create a regression model to determine mean daily bed sediment volumes. The bed sediment regression was applied to mean annual daily discharge for each monitored stream. Values of mean daily bed sediment load mass were calculated from these data. Bed sediment volumes measured during 2012-2013 were applied to mean daily discharge from the 2004-2005 period to obtain comparable loads.

Due to a log jam on the upstream side of the bridge at site 16, bed sediment could not be measured during 2012-2013. Therefore, in 2012-2013 bed loads were estimated for nine of the ten sites estimated in 2004-2005. The largest bed sediment transport rate estimated in 2004-2005 was 5,935 t/yr for Rum Creek site 12. The rate for site 12 in 2012-2013 is virtually unchanged and continued to be the largest (5,343 t/yr). The headwaters of Rum Creek are 2.6 miles south of Tuscaloosa. The upstream portion of the creek flows northwestward through residential and rural areas along U.S. Highway 82. Approximately 0.5 mile south of the city, the stream flows through an area of commercial development before its confluence with Cypress Creek near Kauloosa Avenue. The second, third, and fourth largest bed sediment rates in 2004-2005 were Cypress Creek sites 14 (3,410 t/yr), 13 (2,962 t/yr), and 5 (1,777 t/yr), respectively. Sites 14 and 13 continued to have the second and third largest loads in 2012-2013, but the fourth largest load was estimated at Cribbs Mill Creek site 4, due to the major decrease in bed sediment transport at Cypress Creek site 5. Figure 27 shows decreasing bed sediment loads in the downstream direction for the 2004-2005 and 2012-2013 monitoring periods for Cypress Creek. This was likely caused by sediment deposition in the low gradient reach between sites 13 and 5 and periodic evacuation of deposited sediment during large discharge events that was not measured during the monitoring period. However, monitoring showed that sediment loads in the Cypress Creek watershed were significantly lower during 2012-2013 than during 2004-2005 (fig. 28). As discussed previously, several large construction projects, ongoing during the 2004-2005 project period contributed large amounts of sediment to Cypress Creek. During 2012-2013 many of these disturbed sites were stabilized. Also, in 2004 and 2005, severe stream bank erosion was observed along much of the creek. This continued to be a major source of sediment in 2012 and 2013. Bed

Cypress Creek

Cribbs Mill Creek

Figure 27.—Estimated bed sediment loads for city of Tuscaloosa monitored watersheds for the 2004-2005 and 2012-2013 monitoring periods.

Figure 28.—Bed sediment measured at site 5 during the 2004-2005 and 2012-2013 monitoring periods.

sediment transport rates at all other sites were much less, with the smallest rate estimated at the upstream Cypress Creek site 15 (32 t/yr). Figure 27 portrays the bed sediment transport rates for the 2004-2005 and 2012-2013 monitoring periods.

Whereas total annual bed sediment transport rates indicate the mass of bed sediment moving through a watershed or portion of a watershed, normalized (unit load per unit area, t/mi2/yr) loads permit comparisons of one watershed to another irrespective of differences in area. Normalization of the bed sediment data for the Tuscaloosa stormwater assessment indicates that Cypress Creek site 14 had the largest bedload per square mile of drainage area (2,288) t/mi2/yr). Rum Creek site 12 had the second largest load (960 t/mi2/yr). Cribbs Mill Creek site 17 had the smallest load (19 t/mi2/yr). Normalized loads for all sites are shown in figure 29.

Figure 29.—Estimated normalized bed sediment loads for city of Tuscaloosa monitored watersheds for the 2004-2005 and 2012-2013 monitoring periods.

Total Sediment Loads

The total sediment load transported by a stream is composed of its suspended load and bed load. For streams with sand or gravel beds, the suspended and bed loads were estimated separately and combined. For streams with beds composed of rock or stream beds composed of concrete or limestone boulders, sediment loads are almost totally suspended. In these cases, water samples were collected near the stream bed and were assumed to contain representative volumes of the total sediment load. The suspended sediment loads determined for these sites were considered to be the total sediment loads. The largest total sediment load was estimated at site 13 (21,894 t/yr) for the 2004-2005 monitoring period and at site 6 (16,500 t/yr) for the 2012-2013 monitoring period. On average, during the 2004-2005 monitoring period, bed sediment composed approximately 18 percent (%) of the total sediment load for all monitored watersheds. The largest percentage (49%) was estimated at site 19. During the 2012-2013 monitoring period, bed sediment made up about 23% of the total sediment load for all monitored watersheds. The largest percentage (95%) was estimated at site 14. Suspended and bed sediment contributions to total sediment loads are portrayed in figures 30 and 31.

All sediment transported by project streams is deposited directly into the Black Warrior River, Moody Swamp, or Hurricane Creek. An estimated sediment load of 2,572 t/yr (1,594 cubic yards) was deposited directly into the Black Warrior River during the 2004-2005 monitoring period and about 1,700 t/yr (1,054 cubic yards) during the 2012-2013 monitoring period. Moody Swamp received 8,721 t/yr (5,407 cubic yards) during the 2004-2005 monitoring period and 10,504 t/yr (6,513 cubic yards) during the 2012-2013 monitoring period. Hurricane Creek received 20,145 t/yr (12,490 cubic yards) during the 2004-2005 monitoring period and 16,778 t/yr (10,402 cubic yards) during the 2012-2013 monitoring period. Table 23 gives detailed statistics for total sediment loads for all project sites.

Comparisons of sediment loads from other watersheds are helpful in determining the severity of erosion problems in a watershed of interest. Normalized estimates of total sediment loads from watersheds of the Choctawhatchee River in southeast Alabama, Yellow River in Covington County, Bear Creek in Colbert and Franklin Counties, Black Warrior River at Tuscaloosa, Conecuh River at Gantt and Point A reservoirs in Covington County, D’Olive Creek at Daphne, Dog River at Mobile, and Joes Branch at

Figure 30.—Total sediment loads for monitored sites during the 2004-2005 monitoring period.

Figure 31.—Total sediment loads for monitored sites during the 2012-2013 monitoring period.

Table 23.—Summary statistical data for sediment loads estimated for the

Tuscaloosa storm-water assessment.

| Site | Total sediment loads |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Tons/year |  | Tons/mi2/year |  | Pounds/acre/year |  | Cubic yards /year |  | Cubic yards/mi2/year |  |
|  | 2004-2005 | 2012-2013 | 2004-2005 | 2012-2013 | 2004-2005 | 2012-2013 | 2004-2005 | 2012-2013 | 2004-2005 | 2012-2013 |
| 1 | 60 | 16 | 155 | 41 | 488 | 130 | 37 | 10 | 96 | 25 |
| 2 | 497 | 75 | 564 | 85 | 1,761 | 266 | 307 | 46 | 348 | 53 |
| 3 | 124 | 115 | 56 | 52 | 175 | 162 | 77 | 71 | 35 | 33 |
| 4 | 2,965 | 8,601 | 616 | 1,787 | 4,996 | 14,493 | 1,830 | 5,309 | 989 | 2,869 |
| 5 | 5,135 | 1,713 | 4,892 | 1,632 | 49,146 | 16,395 | 8,417 | 2,808 | 9,708 | 3,239 |
| 6 | 15,479 | 16,500 | 3,395 | 3,619 | 8,735 | 9,311 | 9,555 | 10,185 | 1,725 | 1,839 |
| 7 | 745 | 46 | 424 | 26 | 1,324 | 82 | 460 | 28 | 262 | 16 |
| 8 | 3,921 | 232 | 3,019 | 179 | 9,890 | 585 | 2,420 | 143 | 1,953 | 116 |
| 9 | 339 |  | 780 |  | 2,441 |  | 209 |  | 482 |  |
| 10 | 165 | 8 | 163 | 8 | 508 | 25 | 102 | 5 | 101 | 5 |
| 11 |  | 7 |  | 11 |  | 36 |  | 4 |  | 7 |
| 12 | 17,364 | 14,586 | 2,803 | 2,355 | 8,746 | 7,347 | 10,719 | 9,004 | 1,728 | 1,452 |
| 13 | 21,894 | 1,747 | 7,337 | 585 | 70,535 | 5,628 | 13,515 | 1,078 | 13,933 | 1,111 |
| 14 | 14,931 | 3,597 | 7,230 | 1,742 | 31,302 | 7,541 | 9,217 | 2,221 | 6,182 | 1,490 |
| 15 | 181 | 71 | 215 | 84 | 672 | 264 | 112 | 44 | 133 | 52 |
| 16 | 1,418 | 1,050 | 428 | 317 | 1,328 | 983 | 875 | 648 | 262 | 194 |
| 17 | 1,337 | 1,548 | 449 | 520 | 1,402 | 1,623 | 825 | 955 | 277 | 321 |
| 18 | 237 | 139 | 117 | 69 | 383 | 225 | 146 | 86 | 76 | 45 |
| 19 | 279 | 170 | 241 | 148 | 751 | 458 | 169 | 103 | 149 | 92 |
| 20 | 1,729 | 1,406 | 511 | 416 | 1,628 | 1,324 | 1,064 | 2,052 | 322 | 262 |

Spanish Fort are shown in figure 32 (Cook and Moss, 2007, 2012; Cook and others, 2008) along with the dominant land use in each watershed. These data indicate that the largest sediment loads occur in urban watersheds where land disturbances and high volume/high velocity runoff are common. Figure 32 also shows that urban streams have the lowest percentages of bed sediment, also attributable to high volume/high velocity runoff resulting from impervious surfaces, channelization, and channel armoring.  GSA sediment transport assessments in about 60 watersheds throughout the state indicate that the city of Tuscaloosa urban stream sediment transport rates are among the highest (fig. 32). Figure 32 shows that the smallest sediment loads are in the Yellow River watershed (primarily forested). Streams in the Choctawhatchee River watershed in southeast Alabama and the Bear Creek watershed in northwest Alabama have moderate sediment loads (primarily from row crop agriculture and timber harvesting).  Tributaries to the Gantt and Point A reservoirs in south-central Alabama have large sediment loads, primarily from eroding unpaved roads, and D’Olive Creek sediment is primarily from urban and developing urban areas in the watershed. Figure 32 also shows that sites with the largest sediment loads are from storm-water runoff in the more mature urban watersheds in the city of Tuscaloosa and Dog River in the city of Mobile. Joes Branch site JB7 in Spanish Fort has the largest normalized sediment load by far, originating from a single catastrophic erosional feature.

Figure 32.—Comparisons of estimated sediment loads for selected watersheds with various

dominant land use classifications.

Agriculture

Forest

Urban

Forest with unpaved roads

Transitional

urban

SUMMARY AND CONCLUSIONS

The City of Tuscaloosa storm-water assessment consists of comprehensive physical and chemical analyses of water quality in 10 watersheds in 2004-2005 and 9 watersheds in 2012-2013 and analyses of physical characteristics and sedimentation at 10 additional sites that receive storm-water drainage from the city. More than 400 samples were collected and more than 7,000 water-quality parameters were measured at monitoring sites during the project. The primary physical parameters of concern for storm-water runoff are water temperature, DO concentrations, turbidity, sediment loads, stream discharge and stream flow velocity.

The standard for maximum temperature established by the Alabama Department of Environmental Management for surface water classified as Fish and Wildlife is 32.3º C. The standard was not exceeded in any monitored site during the current project period or in the previous project period.

Turbidity is an indication of the amount of sediment being transported by streams. The highest average turbidity values were measured at sites 4, 5, 12, 13 and 19 with values at sites 12 and 19 more than 1,000 NTUs. The lowest values were measured at sites 7, 14, 16, and 17. Average turbidity in area streams is generally lower in the current study when compared with the previous study with the exception of sites 18, 19, and 20 and average discharge is higher with the exception of sites 2, 3, and 4.

The largest total sediment load was estimated at site 13 (21,894 t/yr) for the 2004-2005 monitoring period and at site 6 (16,500 t/yr) for the 2012-2013 monitoring period. On average, during the 2004-2005 monitoring period, bed sediment composed approximately 18% of the total sediment load for all monitored watersheds. All sediment transported by project streams is deposited directly into the Black Warrior River, Moody Swamp, or Hurricane Creek. An estimated sediment load of 2,572 t/yr (1,594 cubic yards) was deposited directly into the Black Warrior River during the 2004-2005 monitoring period and about 1,700 t/yr (1,054 cubic yards) during the 2012-2013 monitoring period. Moody Swamp received 8,721 t/yr (5,407 cubic yards) during the 2004-2005 monitoring period and 10,504 t/yr (6,513 cubic yards) during the 2012-2013 monitoring period. Hurricane Creek received 20,145 t/yr (12,490 cubic yards) during the 2004-2005 monitoring period and 16,778 t/yr (10,402 cubic yards) during the 2012-2013 monitoring period. Excessive sediment in the monitored watersheds is caused by land disturbance in the urban environment and erosion of stream beds and banks and drainage channels caused by large volumes of runoff from impervious surfaces and excessive stream flow velocities caused by channelized and armored streams and drainage ways.

The primary chemical parameters of concern for water quality are nutrients, metals, nonmetallic inorganic constituents, and organic compounds. The results of monitoring and analyses indicate that nutrients and a small number of metals and nonmetallic inorganic constituents have high concentrations in many of the project streams. The maximum nonpoint-source contaminant criteria for ammonia were not exceeded during the monitoring period. The maximum criteria for nitrate were exceeded at sites 5, 6, 7, and 8 and the criteria for phosphorus were exceeded at all monitored sites.  Sources of excessive amounts of nutrients include runoff from over-fertilization of lawns in residential areas, industrial runoff, and wastewater.

The USEPA compiled national recommended water quality criteria for the protection of aquatic life and human health in surface water for approximately 150 pollutants. The criteria were developed for acute (short-term exposure) and chronic (long-term exposure) concentrations. Many constituents observed in surface water are naturally occurring and originate from the dissolution or erosion of rocks or sediments. In most cases, these constituents occur in relatively small concentrations and have no detrimental effects on the environment or human health. Barium, calcium, potassium, sodium, and strontium are very common in many rock types and are common in aquatic environments. All samples collected during the project contained these constituents in relatively small concentrations.

A number of metallic elements were detected throughout the project area. However, most of these occur naturally in the Pottsville Formation and the Coker Formation. Arsenic, nickel, and zinc occur naturally in metamorphic and igneous rocks and are present in some waters but are much less common than barium, calcium, potassium, sodium, and strontium. Arsenic is used in pesticides and industrial processes. Nickel and zinc are used in metallurgical processes, and zinc is used as a white pigment in paint and rubber. Detections of arsenic and nickel were all below the USEPA criteria. Arsenic and zinc were detected at all sites while nickel was not detected. Zinc was measured at a concentration exceeding the USEPA criteria at site 8.

Lead, usually in small concentrations, is pervasive in the environment. The sources of lead are varied and include industrial waste and atmospheric transport from regional or intercontinental sources. Lead was detected in all sampled watersheds but only exceeded the USEPA criteria at site 8. The source of lead in this watershed is unknown.

Seven inorganic nonmetallic constituents were analyzed from water samples collected at the comprehensive monitoring sites. Boron is occasionally detected in the surface waters of Alabama. Boron is naturally associated with igneous rocks and is present in active volcanic areas. In areas without a natural source, it may originate from cleaning wastes and may be present in sewage and industrial wastes. Boron was detected at sites 3, 4, 6 and 7, although concentrations were relatively small. Chloride, silica, and sulfate were detected at all monitored sites and fluoride was detected at sites 1, 3, 4, and 7. These constituents are common in surface water and usually originate, in the observed range of concentrations, from sediments that underlie the monitored watersheds.

A limited number of organic constituents were analyzed in water samples collected from the monitored sites (comprehensive analytical monitoring sites). They include TOC (discussed previously), phenols, and oil and grease. Concentrations observed are similar to those observed in the 2005 study.

Typical TOC values for natural waters vary from 1 to 10 mg/L. All but two of the TOC values measured in the current project period are near or exceeding the 10 mg/L threshold. Sites 6, 7, 8 and 10 have increased TOC values when compared with 2005 levels while sites 1, 2, 3 and 5 have decreased values. TOC at site 4 remained relatively unchanged.

Phenols are used in the production of phenolic resins, germicides, herbicides, fungicides, pharmaceuticals, dyes, plastics, and explosives. Phenols cause acute and chronic toxicity to freshwater aquatic life. The EPA states that phenol should be limited to 0.3 mg/L in lakes and streams to protect human health from the possible harmful effects of exposure. Phenol concentrations were well above recommended levels in all monitored streams.

Concentrations of oil and grease were determined for water samples collected from sites 1-10. Oil and grease includes fatty matter from animal and vegetable sources and from hydrocarbons of petroleum origin. Concentrations were not detected above the detection limit of 5.0 mg/L

Collection of adequate amounts of comprehensive data is the first step to understanding any hydrogeochemical system. Data collected during the City of Tuscaloosa stormwater assessment answered a number of questions related to the physical and chemical character of stormwater runoff and the effects of this runoff on the urban hydrologic environment in the monitored watersheds. However, a number of watersheds were not evaluated during the project.

Collection of an initial dataset from Moody Swamp should be conducted to assess the current quality of water and to establish a base-line dataset to evaluate future changes of water quality in this very important wetland.

Erosion and sedimentation are major problems in the city. Steps must be taken to enforce regulations related to control of erosion and runoff from construction sites. Runoff from impervious surfaces should be detained to the fullest extent possible to reduce the amount of discharge in urban streams during storm events. Stream channels should be restored to their natural configuration wherever possible, and if stream restoration is not possible, unprotected stream banks should be armored to prevent stream bank erosion.

Excessive concentrations of nutrients appear to be a problem in many areas of the city. An educational program designed to reduce the use of commercial fertilizers should be implemented. Discussions with the University of Alabama may also be helpful. Garbage and debris in streams in the city is a major problem. A program of education and resident involvement to clean up stream channels and flood plains is needed.

Acquisition of the latest high resolution photography and the evaluation and interpretation of this imagery is needed to determine the effects of land use on runoff and environmental quality in the city. These data will indicate patterns of development and amounts of impervious surface that may be a valuable tool for future planning and development in Tuscaloosa.

Additional evaluation of the storm-water runoff and land use in the City of Tuscaloosa is needed to develop a comprehensive understanding of this urban hydrologic system. However, it is essential that the findings from this and future scientific assessments should be utilized to develop and implement best management practices (BMPs) to improve water quality and the overall quality of the urban environment in Tuscaloosa.

REFERENCES CITED

Alabama Department of Environmental Management, 1992, Water-quality criteria for surface waters classified Fish and Wildlife.

Bevans, H. E., Lico, M. S., Lawrence, S. J., 1998, Water quality in the Las Vegas Valley area and Carson and Truckee River Basins, Nevada and California, 1992-96, URL http://water.usgs.gov/pubs/circ/circ1170/gloss.htm accessed April 1, 2005.

City of Tuscaloosa, 2011, Tornado path map, URL http://bubblessoc.net/archives/2011-tuscaloosa-tornado/, accessed August 11, 2013.

Cohn, T. A., Caulder, D. L., Gilroy, E. J., Zynjuk, L. D., and Summers, R. M., 1992, The validity of a simple statistical model for estimating fluvial constituent loads: an impirical study involving nutrient loads entering Chesapeake Bay: Water Resources Research, v. 28, p. 2353-2363.

Cook, M. R., Henderson, W. P., Moss, N. E., Baker, R. M., 2005, City of Tuscaloosa storm-water assessment: Geological Survey of Alabama Open-file Report 0514, 118 p.

Cook, M. R., and Moss, N. E., 2007, Analysis of sediment loading rates and impacts of unpaved roads on selected tributaries to Gantt and Point A Lakes, Covington County, Alabama 2002-2007: Geological Survey of Alabama Open-file Report 0703, 41 p.

Cook, M. R., and Moss, N. E., 2008, Analysis of water quality, sediment loading, biological resources, and impacts of land-use change on the D’Olive and Tiawasee Creek watersheds, Baldwin County, Alabama, 2008: Geological Survey of Alabama Open-file Report 0811, 140 p.

Cook, M. R., and Moss, N. E., 2012, Analysis of discharge and sediment loading rates in tributaries of Dog River in the Mobile metropolitan area: Geological Survey of Alabama Open-file Report 1214, 24 p.

Eaton, A. D., Clesceri, L. S., and Greenberg, A. E., ed., 1995, Standard methods for the examination of water and wastewater, 19th edition: Washington, D. C., American Public Health Association, p. 9-53--9-72.

Geological Survey of Alabama, 2006, Digital geologic map of Alabama, GSA Special Map 220A.

Hem, J. D., 1985, Study and interpretation of the chemical characteristics of natural waters (3rd ed.): U.S. Geological Survey Water Supply Paper no. 2254, 264 p.

Maidment, D. R., ed., 1993, Handbook of hydrology: New York, Mcgraw-Hill Inc., p. 11.37-11.54.

Mays, L. W., ed., 1996, Water resources handbook: New York, Mcgraw-Hill, p. 8.3-8.49.

Osborne, W. E., Szabo, M. W., and Copeland, C. W., 1988, Geologic map of Alabama: Alabama Geological Survey, Special Map 220, Northwest Sheet, scale 1:250,000.

Richards, R. Peter, 1999, Estimation of pollutant loads in rivers and streams: a guidance document for NPS programs: Tiffin, Ohio, Heidelberg College, Water Quality Laboratory.

Schueler, T. R., 1992, Mitigating the adverse impacts of urbanization on streams: a comprehensive strategy for local government.  P. Kimble and T. Schueler, editors, in Watershed Restoration Sourcebook: Metropolitan Washington Council of Governments,  Publication #92701.

___1995, Site planning for urban stream protection: Metropolitan Washington Council of Governments, p. 20.

USEPA, 2002, Watershed Academy Web: Eight tools of watershed protection in developing areas: URL http://www.epa.gov/watertrain/protection/index.html accessed April 1, 2005.

USEPA, 2004, Monitoring and assessing water quality: Fecal bacteria, URL 	http://www.epa/gov/OWOW/monitoring/volunteer/stream/vms511.html accessed 	April 1, 2005.

USEPA, 2005, Water and wastewater security product guide: Chemical sensor – Total organic carbon analyzer, URL http://www.epa.gov/safewater/watersecurity/guide/chemicalsensortotalorganiccarbonanalyzer.html accessed April 1, 2005.

###### Appendix

Comprehensive Lab Results

| Site: | T01 | T02 | T03 | T04 | T05 | T06 | T07 | T08 | T10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Parameter: |  |  |  |  |  |  |  |  |  |
| Ag | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| µg/L |  |  |  |  |  |  |  |  |  |
| 10 |  |  |  |  |  |  |  |  |  |
| Al | 0 | 83 | 0 | 95 | 0 | 64 | 86 | 132 | 0 |
| µg/L |  |  |  |  |  |  |  |  |  |
| 60 |  |  |  |  |  |  |  |  |  |
| Alkalinity | 13 | 18 | 121 | 19 | 19 | 24 | 35 | 6 | 30 |
| mg/L as CaCO3 |  |  |  |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |  |  |  |
| HCO3 | 16 | 22 | 147 | 23 | 23 | 29 | 43 | 7 | 37 |
| mg/L |  |  |  |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |  |  |  |
| CO3 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| mg/L |  |  |  |  |  |  |  |  |  |
| 1 |  |  |  |  |  |  |  |  |  |
| CO2, Free | 8 | 6 | 37 | 3 | 4 | 7 | 9 | 1 | 24 |
| mg/L |  |  |  |  |  |  |  |  |  |
| 1 |  |  |  |  |  |  |  |  |  |
| Arsenic | 1.12 | 1.09 | 1.72 | 1.21 | 0.68 | 0.73 | 0.62 | 0.62 | 0.86 |
| µg/L |  |  |  |  |  |  |  |  |  |
| 0.3 |  |  |  |  |  |  |  |  |  |
| B | 0 | 0 | 197 | 114 | 0 | 63 | 51 | 0 | 0 |
| µg/L |  |  |  |  |  |  |  |  |  |
| 110 |  |  |  |  |  |  |  |  |  |
| Ba | 27.2 | 37.1 | 165.5 | 57.8 | 57.3 | 72.7 | 74.8 | 71.1 | 75.2 |
| µg/L |  |  |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |  |  |
| Be | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| µg/L |  |  |  |  |  |  |  |  |  |
| 1 |  |  |  |  |  |  |  |  |  |
| Br | 0.00 | 0.00 | 0.24 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| mg/L |  |  |  |  |  |  |  |  |  |
| 0.05 |  |  |  |  |  |  |  |  |  |
| Ca | 6.88 | 8.41 | 43.20 | 9.52 | 9.11 | 11.30 | 15.80 | 6.38 | 11.80 |
| mg/L |  |  |  |  |  |  |  |  |  |
| 0.4 |  |  |  |  |  |  |  |  |  |
| Cd | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.1 | 0.0 |
| µg/L |  |  |  |  |  |  |  |  |  |
| 0.1 |  |  |  |  |  |  |  |  |  |
| Cl | 2.83 | 0.85 | 6.72 | 2.04 | 2.38 | 5.00 | 5.08 | 3.27 | 4.34 |
| mg/L |  |  |  |  |  |  |  |  |  |
| 0.05 |  |  |  |  |  |  |  |  |  |

| Site: | T01 | T02 | T03 | T04 | T05 | T06 | T07 | T08 | T10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Parameter: |  |  |  |  |  |  |  |  |  |
| CN | 0.000 | 0.000 | 0.0046 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| mg/L |  |  |  |  |  |  |  |  |  |
| 0.003 |  |  |  |  |  |  |  |  |  |
| Co | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| µg/L |  |  |  |  |  |  |  |  |  |
| 7 |  |  |  |  |  |  |  |  |  |
| Cr | 0.00 | 0.57 | 0.00 | 0.64 | 0.61 | 0.00 | 0.00 | 1.22 | 0.32 |
| µg/L |  |  |  |  |  |  |  |  |  |
| 0.6 |  |  |  |  |  |  |  |  |  |
| Cr6+ | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| µg/L |  |  |  |  |  |  |  |  |  |
| 5.0 |  |  |  |  |  |  |  |  |  |
| Cs | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| µg/L |  |  |  |  |  |  |  |  |  |
| 4.0 |  |  |  |  |  |  |  |  |  |
| Cu | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| µg/L |  |  |  |  |  |  |  |  |  |
| 5.0 |  |  |  |  |  |  |  |  |  |
| DO | 11.2 | 9.3 | 10.7 | 8.2 | 13.7 | 11.6 | 8.73 | 11.1 | 11.8 |
| mg/L |  |  |  |  |  |  |  |  |  |
| Dissolved Solids, calculated | 28 | 28 | 154 | 35 | 34 | 49 | 67 | 29 | 53 |
| mg/L |  |  |  |  |  |  |  |  |  |
| F | 0.074 | 0.000 | 0.074 | 0.069 | 0.000 | 0.092 | 0.000 | 0.000 | 0.000 |
| mg/L |  |  |  |  |  |  |  |  |  |
| 0.020 |  |  |  |  |  |  |  |  |  |
| Fe | 38.9 | 43.0 | 332.0 | 109.0 | 137.0 | 211.0 | 311.0 | 356.0 | 204.0 |
| µg/L |  |  |  |  |  |  |  |  |  |
| 3.0 |  |  |  |  |  |  |  |  |  |
| Hardness | 20 | 24 | 129 | 28 | 28 | 38 | 55 | 22 | 39 |
| mg/L as CaCO3 |  |  |  |  |  |  |  |  |  |
| Hg | 0.000 | 0.028 | 0.031 | 0.037 | 0.000 | 0.000 | 0.026 | 0.005 | 0.000 |
| µg/L |  |  |  |  |  |  |  |  |  |
| 0.010 |  |  |  |  |  |  |  |  |  |
| I | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| mg/L |  |  |  |  |  |  |  |  |  |
| 0.2 |  |  |  |  |  |  |  |  |  |
| K | 2.54 | 2.53 | 5.76 | 2.38 | 1.83 | 2.39 | 2.54 | 2.35 | 2.97 |
| mg/L |  |  |  |  |  |  |  |  |  |
| 0.5 |  |  |  |  |  |  |  |  |  |
| Li | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| µg/L |  |  |  |  |  |  |  |  |  |
| 5 |  |  |  |  |  |  |  |  |  |

| Site: | T01 | T02 | T03 | T04 | T05 | T06 | T07 | T08 | T10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Parameter: |  |  |  |  |  |  |  |  |  |
| Mg | 0.656 | 0.697 | 5.170 | 1.110 | 1.210 | 2.450 | 3.730 | 1.500 | 2.290 |
| mg/L |  |  |  |  |  |  |  |  |  |
| 0.040 |  |  |  |  |  |  |  |  |  |
| Mn | 18.3 | 18.5 | 419.0 | 42.7 | 51.9 | 228.0 | 171.0 | 87.8 | 122.0 |
| µg/L |  |  |  |  |  |  |  |  |  |
| 2.0 |  |  |  |  |  |  |  |  |  |
| Mo | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| µg/L |  |  |  |  |  |  |  |  |  |
| 20 |  |  |  |  |  |  |  |  |  |
| Na | 1.00 | 0.94 | 10.20 | 1.47 | 1.40 | 2.37 | 3.13 | 2.16 | 3.67 |
| mg/L |  |  |  |  |  |  |  |  |  |
| 0.05 |  |  |  |  |  |  |  |  |  |
| NH3_N | 0.270 | 0.000 | 0.093 | 0.150 | 0.103 | 0.090 | 0.073 | 0.000 | 0.202 |
| mg/L |  |  |  |  |  |  |  |  |  |
| 0.020 |  |  |  |  |  |  |  |  |  |
| Ni | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| µg/L |  |  |  |  |  |  |  |  |  |
| 10 |  |  |  |  |  |  |  |  |  |
| NO2_N | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| mg/L |  |  |  |  |  |  |  |  |  |
| 0.01 |  |  |  |  |  |  |  |  |  |
| NO2_NO2 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| mg/L |  |  |  |  |  |  |  |  |  |
| 0.03 |  |  |  |  |  |  |  |  |  |
| NO3_N | 0.447 | 0.180 | 0.144 | 0.343 | 0.574 | 0.528 | 0.881 | 0.711 | 0.433 |
| mg/L |  |  |  |  |  |  |  |  |  |
| 0.020 |  |  |  |  |  |  |  |  |  |
| NO3_NO3 | 1.98 | 0.80 | 0.64 | 1.52 | 2.54 | 2.34 | 3.90 | 3.15 | 1.92 |
| mg/L |  |  |  |  |  |  |  |  |  |
| 0.09 |  |  |  |  |  |  |  |  |  |
| Nox_N | 0.447 | 0.180 | 0.144 | 0.343 | 0.574 | 0.528 | 0.881 | 0.711 | 0.433 |
| mg/L |  |  |  |  |  |  |  |  |  |
| 0.020 |  |  |  |  |  |  |  |  |  |
| Nox_NO3 | 1.98 | 0.80 | 0.64 | 1.52 | 2.54 | 2.34 | 3.90 | 3.15 | 1.92 |
| mg/L |  |  |  |  |  |  |  |  |  |
| 0.09 |  |  |  |  |  |  |  |  |  |
| P | 0.144 | 0.315 | 0.098 | 0.236 | 0.166 | 0.140 | 0.103 | 0.133 | 0.684 |
| mg/L |  |  |  |  |  |  |  |  |  |
| 0.010 |  |  |  |  |  |  |  |  |  |
| PO4_P | 0.05 | 0.13 | 0.00 | 0.05 | 0.00 | 0.00 | 0.00 | 0.00 | 0.04 |
| mg/L |  |  |  |  |  |  |  |  |  |
| 0.02 |  |  |  |  |  |  |  |  |  |
| Pb | 0.71 | 0.73 | 0.63 | 0.71 | 0.56 | 0.65 | 1.20 | 2.76 | 0.86 |
| µg/L |  |  |  |  |  |  |  |  |  |
| 2.0 |  |  |  |  |  |  |  |  |  |

| Site: | T01 | T02 | T03 | T04 | T05 | T06 | T07 | T08 | T10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Parameter: |  |  |  |  |  |  |  |  |  |
| Rb | 0.0 | 0.0 | 8.3 | 3.5 | 2.5 | 2.8 | 3.0 | 2.0 | 3.0 |
| µg/L |  |  |  |  |  |  |  |  |  |
| 2.0 |  |  |  |  |  |  |  |  |  |
| S2-(sulfide) | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| mg/L |  |  |  |  |  |  |  |  |  |
| 0.02 |  |  |  |  |  |  |  |  |  |
| Sb | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| µg/L |  |  |  |  |  |  |  |  |  |
| 3.0 |  |  |  |  |  |  |  |  |  |
| Se | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| µg/L |  |  |  |  |  |  |  |  |  |
| 2.0 |  |  |  |  |  |  |  |  |  |
| SiO2 | 0.88 | 1.20 | 7.15 | 1.56 | 1.77 | 4.01 | 4.00 | 3.97 | 3.06 |
| mg/L |  |  |  |  |  |  |  |  |  |
| 0.05 |  |  |  |  |  |  |  |  |  |
| Sn | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| µg/L |  |  |  |  |  |  |  |  |  |
| 50 |  |  |  |  |  |  |  |  |  |
| SO4 | 3.55 | 1.88 | 2.78 | 4.23 | 2.48 | 4.20 | 7.61 | 2.85 | 4.64 |
| mg/L |  |  |  |  |  |  |  |  |  |
| 0.06 |  |  |  |  |  |  |  |  |  |
| Sr | 13.8 | 19.1 | 163.0 | 17.1 | 15.5 | 21.4 | 32.7 | 19.3 | 20.2 |
| µg/L |  |  |  |  |  |  |  |  |  |
| 1.0 |  |  |  |  |  |  |  |  |  |
| Solids, Total Dissolved | 50 | 46 | 171 | 52 | 49 | 76 | 93 | 62 | 80 |
| Mg/L |  |  |  |  |  |  |  |  |  |
| 10 |  |  |  |  |  |  |  |  |  |
| Ti | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| µg/L |  |  |  |  |  |  |  |  |  |
| 4.0 |  |  |  |  |  |  |  |  |  |
| TKN | 4.70 | 1.64 | 1.15 | 2.69 | 2.10 | 1.59 | 1.32 | 1.14 | 17.50 |
| mg/L |  |  |  |  |  |  |  |  |  |
| 0.10 |  |  |  |  |  |  |  |  |  |
| Tl | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| µg/L |  |  |  |  |  |  |  |  |  |
| 2.0 |  |  |  |  |  |  |  |  |  |
| V | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| µg/L |  |  |  |  |  |  |  |  |  |
| 4.0 |  |  |  |  |  |  |  |  |  |
| Zn | 39.9 | 52.0 | 52.4 | 109.0 | 75.2 | 66.4 | 89.9 | 204.0 | 81.4 |
| µg/L |  |  |  |  |  |  |  |  |  |
| 4.0 |  |  |  |  |  |  |  |  |  |

| Site: | T01 | T02 | T03 | T04 | T05 | T06 | T07 | T08 | T10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Parameter: |  |  |  |  |  |  |  |  |  |
| BOD5 | 8.0 | 7.0 | 4.0 | 7.0 | 7.5 | 6.9 | 7.9 | 5.3 | 5.4 |
| mg/L |  |  |  |  |  |  |  |  |  |
| 0.1 |  |  |  |  |  |  |  |  |  |
| COD | 111 | 130 | 96 | 54 | 114 | 106 | 93 | 92 | 496 |
| mg/L |  |  |  |  |  |  |  |  |  |
| 30 |  |  |  |  |  |  |  |  |  |
| Phenolics | 10.3 | 10.9 | 4.2 | 10.8 | 7.1 | 12.3 | 10.6 | 6.0 | 29.8 |
| ug/L |  |  |  |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |  |  |  |
| Total Organic Carbon | 9.56 | 11.60 | 5.55 | 10.80 | 8.05 | 9.90 | 10.00 | 9.18 | 21.50 |
| mg/L |  |  |  |  |  |  |  |  |  |
| 0.40 |  |  |  |  |  |  |  |  |  |
| Hexane Extractable Material | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| mg/L |  |  |  |  |  |  |  |  |  |
| 5.0 |  |  |  |  |  |  |  |  |  |
| Silica-gel Treated Hexane Extractable Material | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| mg/L |  |  |  |  |  |  |  |  |  |
| 5.0 |  |  |  |  |  |  |  |  |  |
| Surfactants (MBAS) | 0.050 | 0.082 | 0.005 | 0.014 | 0.023 | - | 0.044 | 0.024 | 0.021 |
| mg/L |  |  |  |  |  |  |  |  |  |
| 0.40 |  |  |  |  |  |  |  |  |  |
| Total Coliform Bacteria | 155,310 | 464,000 | 43,250 | 241,960 | 141,360 | 104,620 | 98,040 | 173,290 | 4,106,000 |
| cfu/100 mL |  |  |  |  |  |  |  |  |  |
| 1 |  |  |  |  |  |  |  |  |  |
| E. coli Bacteria | 5,040.0 | 27,550.0 | 1,299.7 | 8,390.0 | 7,630.0 | 2,419.6 | 4,040.0 | 6,200.0 | 19,560.0 |
| cfu/100 mL |  |  |  |  |  |  |  |  |  |
| 1 |  |  |  |  |  |  |  |  |  |

###### Appendix

Field Analytical Lab Results

| Site | Date | Time | Discharge | Conductivity | pH | TDS calculated from conductivity | Solids, Total Suspended | Temp | Turbidity |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | cfs | µS/cm | units | mg/L | mg/L | °C | NTU |
|  |  |  |  |  | -- |  | 4 |  | 1 |
| T01 | 12/10/2012 | 9:19 | 19.2 | 51 | 5.9 | 33 | 16 | 18 | 34 |
| T01 | 1/14/2013 | 12:45 | 2.63 | 140 | 6.1 | 91 | 4 | 12 | 33 |
| T01 | 1/30/2013 | 8:00 | 85.3 | 14 | 6.0 | 9 | 124 | 18 | 129 |
| T01 | 2/11/2013 | 12:50 | 5.14 | 50 | 6.1 | 33 | 13 | 14 | 35 |
| T01 | 4/11/2013 | 16:00 | 23 | 46 | 6.5 | 30 | 45 | 21 | 63 |
| T02 | 12/20/2012 | 13:30 | 0.32 | 129 | 7.6 | 84 | 12 | 15 | 73 |
| T02 | 1/14/2013 | 12:10 | 1.14 | 92 | 6.2 | 60 | 8 | 11 | 35 |
| T02 | 1/30/2013 | 8:35 | 27.3 | 29 | 6.0 | 19 | 62 | 16 | 220 |
| T02 | 2/11/2013 | 12:30 | 14 | 57 | 6.1 | 37 | 29 | 14 | 83 |
| T02 | 4/11/2013 | 16:25 | 38 | 50 | 6.8 | 33 | 76 | 20 | 108 |
| T03 | 12/20/2012 | 13:30 | 1.34 | 332 | 6.6 | 216 | 7 | 14 | 29 |
| T03 | 1/14/2013 | 11:45 | 0.96 | 174 | 5.9 | 113 | 12 | 11 | 25 |
| T03 | 1/30/2001 | 8:45 | 1.19 | 200 | 6.1 | 130 | 23 | 16 | 71 |
| T03 | 2/11/2013 | 11:50 | 0.84 | 138 | 5.6 | 90 | 10 | 14 | 19 |
| T03 | 4/11/2013 | 17:00 | 1.3 | 226 | 6.8 | 147 | 15 | 19 | 35 |
| T04 | 12/17/2012 | 8:20 | 29.2 | 121 | 5.7 | 79 | 10 | 16 | 66 |
| T04 | 1/14/2013 | 8:30 | 52.6 | 143 | 5.5 | 93 | 54 | 14 | 14 |
| T04 | 1/30/2013 | 9:00 | - | 32 | 6.6 | 21 | 425 | 16 | 955 |
| T04 | 2/11/2013 | 13:45 | 176.9 | 83 | 6.1 | 54 | 859 | 14 | 112 |
| T04 | 3/11/2013 | 10:04 | - | 61 | 5.4 | 40 | 183 | 15 | 175 |
| T04 | 4/4/2013 | 13:30 | 27.7 | 117 | 6.3 | 76 | 0 | 14 | 21 |
| T04 | 4/11/2013 | 17:25 | - | 64 | 7.1 | 42 | 167 | 20 | 173 |
| T05 | 12/17/2012 | 9:00 | 11.5 | 92 | 6.2 | 60 | 8 | 15 | 28 |
| T05 | 1/14/2013 | 9:35 | 104.6 | 58 | 5.9 | 38 | 146 | 11 | 122 |
| T05 | 1/30/2013 | 9:30 | 156 | 28 | 6.5 | 18 | 1,330 | 16 | 989 |
| T05 | 2/11/2013 | 15:15 | 83.7 | 58 | 6.6 | 38 | 68 | 13 | 58 |
| T05 | 3/11/2013 | 10:17 | - | 47 | 5.3 | 31 | 966 | 13 | 551 |
| T05 | 4/4/2013 | 11:40 | 12.7 | 91 | 6.2 | 59 | 7 | 13 | 27 |
| T05 | 4/11/2001 | 17:45 | - | 54 | 7.0 | 35 | 129 | 19 | 117 |
| T06 | 12/20/2012 | 12:10 | - | 63 | 6.3 | 41 | 90 | 14 | 236 |
| T06 | 1/14/2013 | 10:40 | - | 62 | 5.6 | 40 | 57 | 12 | 80 |
| T06 | 1/30/2013 | 11:00 | - | 30 | 6.6 | 20 | 144 | 15 | 277 |
| T06 | 2/11/2013 | 10:20 | 43.4 | 59 | 5.5 | 38 | 40 | 13 | 56 |
| T06 | 4/11/2013 | 18:20 | - | 77 | 6.8 | 50 | 214 | 19 | 159 |
| T07 | 12/20/2012 | 11:40 | 4.88 | 73 | 6.3 | 47 | 139 | 14 | 398 |
| T07 | 1/14/2013 | 9:50 | 5.06 | 96 | 5.3 | 62 | 11 | 13 | 34 |
| T07 | 1/30/2013 | 11:20 | 30.5 | 47 | 6.6 | 31 | 107 | 15 | 305 |
| T07 | 2/11/2013 | 9:45 | 5.2 | 84 | 5.3 | 55 | 11 | 14 | 7 |
| T07 | 4/11/2013 | 18:45 | 33 | 97 | 6.9 | 63 | 136 | 18 | 143 |

| Site | Date | Time | Discharge | Conductivity | pH | TDS calculated from conductivity | Solids, Total Suspended | Temp | Turbidity |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | cfs | µS/cm | units | mg/L | mg/L | °C | NTU |
|  |  |  |  |  | -- |  | 4 |  | 1 |
| T08 | 12/20/2012 | 10:35 | 7.38 | 47 | 6.1 | 31 | 98 | 16 | 279 |
| T08 | 1/14/2013 | 9:10 | 6.92 | - | - | - | - | - | - |
| T08 | 1/30/2013 | 11:40 | 84 | 40 | 6.7 | 26 | 174 | 16 | 275 |
| T08 | 2/11/2013 | 9:00 | 5.66 | 50 | 5.4 | 33 | 29 | 15 | 28 |
| T08 | 4/11/2013 | 19:15 | 10 | 48 | 6.9 | 31 | 77 | 17 | 118 |
| T10 | 12/20/2012 | 8:30 | 37.4 | 24 | 5.7 | 16 | 330 | 17 | 284 |
| T10 | 1/14/2013 | 13:15 | 6.58 | 105 | 6.2 | 68 | 20 | 12 | 42 |
| T10 | 1/30/2013 | 14:00 | 17.5 | 79 | 6.8 | 51 | 0 | 15 | 92 |
| T10 | 2/11/2013 | 13:10 | 14.7 | 57 | 6.3 | 37 | 31 | 13 | 42 |
| T10 | 4/11/2013 | 15:15 | 50 | 58 | 6.4 | 38 | 733 | 21 | 179 |
| T11 | 12/10/2012 | 9:49 | 30.3 | 60 | 6.0 | 39 | 39 | 17 | 82 |
| T11 | 1/14/2013 | 12:20 | 8.37 | 98 | 6.1 | 64 | 14 | 11 | 30 |
| T11 | 1/30/2013 | 8:20 | 130 | 70 | 5.9 | 46 | 332 | 17 | 409 |
| T11 | 2/11/2013 | 12:10 | 8.22 | 88 | 5.8 | 57 | 34 | 13 | 42 |
| T12 | 12/17/2012 | 9:50 | 11.5 | 95 | 6.3 | 62 | 635 | 16 | 78 |
| T12 | 1/14/2013 | 8:50 | 57.6 | 79 | 5.5 | 51 | 201 | 13 | 315 |
| T12 | 1/30/2013 | 9:15 | 180 | 40 | 6.6 | 26 | 1,710 | 15 | 999 |
| T12 | 2/11/2013 | 14:30 | 107 | 79 | 6.1 | 51 | 673 | 13 | 325 |
| T12 | 3/11/2013 | 10:30 | 206 | 32 | 5.3 | 21 | 158 | 13 | 18 |
| T12 | 4/4/2013 | 12:30 | 21.6 | 81 | 6.2 | 53 | 28 | 13 | 74 |
| T13 | 12/20/2012 | 13:30 | 9.72 | 58 | 6.3 | 38 | 53 | 15 | 169 |
| T13 | 1/14/2013 | 9:05 | 21.1 | 75 | 5.7 | 49 | 99 | 12 | 126 |
| T13 | 1/30/2013 | 9:45 | 5 | 28 | 6.5 | 18 | 1,440 | 16 | 506 |
| T13 | 2/12/2013 | 11:10 | 18.3 | 88 | 5.2 | 57 | 180 | 12 | 10 |
| T13 | 3/11/2013 | 11:30 | 103 | 34 | 5.4 | 22 | 523 | 12 | 19 |
| T13 | 4/4/2013 | 10:30 | 11 | 83 | 6.0 | 54 | 6 | 13 | 28 |
| T14 | 12/10/2012 | 16:40 | 13.6 | 63 | 6.3 | 41 | 36 | 15 | 80 |
| T14 | 1/14/2013 | 9:50 | 31 | 66 | 6.0 | 43 | 126 | 12 | 288 |
| T14 | 1/30/2013 | 10:00 | 65.1 | 31 | 6.5 | 20 | 343 | 15 | 417 |
| T14 | 2/12/2013 | 11:50 | 17.7 | 62 | 5.3 | 40 | 26 | 12 | 10 |
| T14 | 3/11/2013 | 12:07 | 66 | 36 | 5.4 | 23 | 69 | 12 | 9 |
| T14 | 4/4/2013 | 9:50 | 8.29 | 76 | 5.8 | 49 | 8 | 13 | 29 |
| T15 | 12/10/2012 | 16:10 | 7 | 70 | 6.3 | 46 | 39 | 15 | 35 |
| T15 | 1/14/2013 | 10:10 | 15.8 | 62 | 6.1 | 40 | 33 | 13 | 73 |
| T15 | 1/30/2013 | 10:35 | 15.9 | 56 | 6.6 | 36 | 91 | 14 | 59 |
| T15 | 2/12/2013 | 12:00 | 7.15 | 55 | 5.2 | 36 | 17 | 11 | 10 |
| T15 | 3/11/2013 | 12:35 | 25 | 52 | 5.6 | 34 | 39 | 12 | 10 |
| T15 | 4/4/2013 | 8:50 | 4.23 | 72 | 5.6 | 47 | 17 | 15 | 15 |

| Site | Date | Time | Discharge | Conductivity | pH | TDS calculated from conductivity | Solids, Total Suspended | Temp | Turbidity |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | cfs | µS/cm | units | mg/L | mg/L | °C | NTU |
|  |  |  |  |  | -- |  | 4 |  | 1 |
| T16 | 12/10/2012 | 15:40 | 32.4 | 85 | 6.3 | 55 | 143 | 15 | 160 |
| T16 | 1/14/2013 | 10:30 | 55 | 79 | 6.1 | 51 | 608 | 12 | 234 |
| T16 | 1/30/2013 | 12:20 | 48.4 | 56 | 6.6 | 36 | 85 | 15 | 227 |
| T16 | 2/12/2013 | 14:30 | 41.4 | 97 | 5.7 | 63 | 927 | 11 | 7 |
| T16 | 3/11/2013 | 13:20 | 62 | 50 | 5.6 | 33 | 545 | 12 | 11 |
| T17 | 12/10/2012 | 15:00 | 17.9 | 86 | 6.4 | 56 | 27 | 16 | 90 |
| T17 | 1/14/2013 | 10:40 | 20.2 | 52 | 6.3 | 34 | 126 | 11 | 194 |
| T17 | 1/30/2013 | 12:40 | 33.7 | 52 | 6.6 | 34 | 84 | 15 | 161 |
| T17 | 2/12/2013 | 15:20 | 49.5 | 56 | 5.8 | 36 | 97 | 11 | 10 |
| T17 | 3/11/2013 | 14:03 | 46 | 54 | 5.6 | 35 | 57 | 12 | 3 |
| T17 | 4/4/2013 | 14:30 | 5.9 | 113 | 6.4 | 73 | 0 | 14 | 15 |
| T18 | 12/10/2012 | 14:40 | 24.4 | 130 | 6.2 | 85 | 30 | 17 | 155 |
| T18 | 1/14/2013 | 9:15 | 36.9 | 96 | 5.3 | 62 | 48 | 13 | 38 |
| T18 | 1/14/2013 | 11:10 | - | 78 | 5.6 | 51 | 79 | 11 | 182 |
| T18 | 1/30/2013 | 13:00 | 39.6 | 66 | 6.6 | 43 | 58 | 15 | 122 |
| T18 | 2/11/2013 | 11:10 | 61.4 | 74 | 5.7 | 48 | 64 | 15 | 108 |
| T19 | 12/20/2012 | 8:50 | 14.3 | 12 | 5.6 | 8 | 4,110 | 16 | 1120 |
| T19 | 1/14/2013 | 11:15 | 7 | 47 | 6.3 | 31 | 51 | 12 | 79 |
| T19 | 1/30/2013 | 13:20 | 8.46 | 47 | 7.0 | 31 | 101 | 14 | 92 |
| T19 | 2/12/2013 | 16:05 | 8.27 | 48 | 5.9 | 31 | 91 | 11 | 10 |
| T19 | 3/11/2013 | 14:38 | 16 | 43 | 5.6 | 28 | 41 | 12 | 10 |
| T20 | 12/20/2012 | 10:00 | 60 | 57 | 6.0 | 37 | 255 | 15 | 481 |
| T20 | 1/14/2013 | 11:00 | 10 | 93 | 6.1 | 60 | 47 | 12 | 155 |
| T20 | 1/30/2013 | 13:35 | 34.9 | 70 | 6.7 | 46 | 117 | 14 | 249 |
| T20 | 2/11/2013 | 12:50 | 24.5 | 88 | 5.5 | 57 | 151 | 14 | 183 |
| T20 | 3/11/2013 | 15:00 | 60 | 55 | 5.7 | 36 | 296 | 12 | 47 |